Scope of Work:

The Contractor shall provide all labor and materials in order to replace 12 windows for the Louisiana Department of Military Affairs (DMA).

Location:

DMA LANG Natchitoches Readiness Center

Construction and Facilities Management Office

500 Fairgrounds Rd.

Natchitoches, LA 71055

## Specifications:

  - Site work and demolition.

    - The Contractor shall protect all paved surfaces on location for dumpsters with ¾” plywood as applicable.

    - The Contractor shall complete any required selective demolition to complete the repairs.

  - The Contractor shall remove the existing 12 windows on the front of the building and repair any necessary damage to the permanent brick prior to the installation of replacement windows. (See image on page 3)

  - The Contractor shall install 12 Aluminum Hurricane resistant windows and Glazing.

4.    The Contractor shall install 12 Oldcastle Series FG5751T thermal Stormax (or equivalent) 2 ½’’ x 5’’ framing up to 40 ¾” W x 49 ¾” H, glazed with 1-5/16” Storm glass made with Solarban 70 Gray two tempered outboard lite, ½’ bronze anodized spacer, two layers of 1/4'’ clear heat strengthened glass with proprietary Storm glass interlayer.

## General Conditions

  - The Contractor shall be responsible for field verifying all dimensions and existing site conditions of work to be performed. The Contractor shall inspect and verify information provided herein on existing materials and equipment, etc. is correct and inform Project Manager (PM) of any errors or questions prior to commencement of work.

  - The Contractor shall make the building, and any sensitive items secure at the end of each workday by any means necessary to secure same and by the approval of the unit Point of Contact (POC).

  - The Contractor shall take all precautions necessary to prevent damages when making the repairs. This includes movement and parking of equipment on grass or unimproved areas surrounding site, as well as surrounding walls in stairwell areas.

  - All work must comply with all applicable State and Local Safety a Environmental requirements. Occupational Safety and Health Administration (OSHA), National Electrical Code (NEC), Parish, City and State regulation will be required. Contractor will supply all employees with PPE.

  - The Contractor shall call LA 1-Call and all utility companies prior to digging (If necessary).

  - The Contractor shall warranty their workmanship, repairs, and installations for a minimum of a one-year period.

## III. Special Conditions:

- Hours of operation Tuesday- Friday, 7:00 AM to 5:30 PM and closed on all recognized state and federal holidays.

- The Contractor shall contact, prior to mobilization and demobilization, the Project Manager for staging and for satisfactory completion.

- The Contractor is responsible for providing any storage containers. If an onsite storage area is required, the Contractor must confirm a storage location for any materials, fixtures, equipment, trailers, etc. stored on site during the term length of the contract. The location will be approved by the Project Manager during the pre- construction meeting.

- The Contractor shall be responsible for disposal and storage of all materials being removed or installed throughout the project.

- The Contractor shall provide warranty information for the materials used in completing this project.

- The Contractor shall provide a construction work schedule (Gnant like) 48 hours prior to start of work detailing each task of work.

- All specification sheets and submittals must be pre-approved by the Project Manager prior to ordering.

- Project shall be completed 60 days after the issuance of the Notice to Proceed.

- Weather Days are included in the contract time.

## III. Contact information:

- The Project Manager (PM) for the project is Darren Thomas, cell phone

(318) 450-7536 or email darren.k.thomas.mil@army.mil

- The Point of Contact (POC) at the facility job site is SSG Justin Smith, phone (318)-663-0356.

## Project Manager

Darren Thomas

Cell: +1 (318) 450-7536

Email: darren.k.thomas.mil@army.mil JFHQ – CFMO

100 Louisiana BLVD

Minden LA 71055