Scope of Work:

The Contractor shall furnish all labor, materials, tools and equipment necessary to pour a 2” compressed asphalt over stone patched area of the parking lot of the Joint Emergency Services Training Center (JESTC) Building for the Louisiana Department of Public Safety (DPS).

Location:

JESTC

1400 W Irene Road

Zachery, LA 70791

Site Visit Contact:

Richard Perry

225-978-8287

Specifications:

- All work shall meet National Asphalt Pavement Association industry standards and practices.

- Finished asphalt surfaces shall be smooth-surfaced with no voids, rough or un-compacted surfaces.  Any finished asphalt surface that is not smooth-surfaced shall be removed and replaced with new asphalt by the Contractor.

- The Contractor shall remove all dirt, loose rocks and other loose debris before overlaying.

- Areas shall be dry and clean before tack and asphalt is applied by the Contractor.

- All transitions shall be milled where joined to existing roadways and driveways to maintain a 2” compacted thickness with a smooth and even transition by the Contractor.

- The five red areas (125 square yards) on the attachment shall be dug out, cleaned, tacked and leveled with 6” of asphalt by the Contractor.

- The green area (3,175 square yards) on the attachment shall be milled 2", a tack coating (liquid tar) SS18H or equivalent applied and then overlaid with 2” compacted thickness asphalt by the Contractor.

Specify Coating Bidding: ______________________

- The dark blue area (120 square yards) on the attachment shall be dug out, cleaned, tacked and leveled with 10" of asphalt by the Contractor.

Planning and Scheduling:

- The Contractor shall also include mobilization and demobilization costs.

- The Contractor shall be responsible for removal of any and all waste material generated by this project.