Tractor Truck Specifications

Model Required:

Peterbilt Model 579 Tractor with 44-inch Low Roof Sleeper,

Clone of Chassis 747906

These are the minimum specifications for the vehicle described. Where a brand/model is referenced for an option listed below, an equal will be considered.

- Rear Mudflaps and Coil Hangers

- Polyarmor Black Quarter Fenders

- JOST JSK37 Fifth Wheel, Light Weight Top Plate

- 7.6in – 8-in Fifth Wheel Height with 18" Air Slide

- Frame Mounted Deckplate and Access Pkg, RH Side

- PHP10 Iron Preset PLUS Hubs - Front and Rear

- Bendix Air Cam Front HP Drum Brakes 16.5x5

- Gusseted Cam Brackets, Steer Axle - Front and Rear

- Heavy Duty Cam Bushings, Steer Axle - Front and Rear

- Anti-Lock Braking System (ABS) 6S6M

- Omit SmarTire Tire Pressure Monitoring

- Bendix Air Cam Rear Drum Brakes 16.5x7 with Automatic Slack Adjusters & Outboard Mounted Brake Drums.

- Ratio 3.36 Rear Axle

- Peterbilt Low Low Air Leaf 40,000 lb, 52-in Axle Spacing, Light Weight.

- Dash mounted dump switch with indicator light for suspension.

- PACCAR MX-13 455 @1600 GV@2200 1650@900 (2024 Emissions)

- Enable Preferred EIST Settings

- Enable EIST Ambient Temp Overrule

- Typical Operating Speed 62 MPH

- MX Retarder State Cruise Control On (Manual Mode Engine Parameter)

- Powertrain Optimized for Balance

- EPA Emission Warranty

- 3 PACCAR Premium 12V Starting Batteries 3000 CCA

- Crank Driven Fan Clutch MX-11/MX-13

- Exhaust Single RH Side of Cab DPF/SCR right-hand Under Cab.

- 18" Ht, 5" Dia Chrome, Clear Coat Curved Tip Standpipe(s)

- Muffler Guard/Fairing Stainless Steel Half Round

- TX-12 Automated Transmission

- RPL20 Driveline lnteraxle

- RPL25 SD Driveline, 1 Midship Bearing

- Differential Lock and Interaxle lock

- Pull Cords All Air Tanks

- Horizontal AE Mounting Bar Double Spring

- Omit Trailer Hand Valve

- 12' AE Lines Coiled, (2) 7-Way Plugs BOC/BOS socket AE Connections Without Hose Tenna

- FF: BR 14ply 295/75R22.5 R268 Ecopia HR 225

- FF: Accu-Lite Steel Armor 51487PK 22.5X8.25 PHP10-5 hand holes.

- 26" Aluminum 100 Gallon Fuel Tank LH BOC Paddle handle filler cap with threadless filler neck

- Location LH BOC 100 Gallon

- DEF To Fuel Ratio 2:1 Or Greater

- RH Tank None Furnished

- LH Sleeper Baggage Door

- RH Painted Sleeper Vent

- LH Sleeper Side Tilting Window

- Aero Rear View Mirror Housing - Body Color

- Body Color – Northsea Gray L2304

- ConcertClass, AM/FM, Weatherband, 3.5 Aux

- Two Speaker Package For Cab

- Two Speaker Package For Sleeper

- Radio Mute When In Reverse

- Bluetooth Phone and Audio

- USB Port

- CB Terminal Wiring, Microphone Clip, Speaker F /0 CB radio mounted center of header.

- One Antenna for Factory Installed or F/O CB Mounted LH cowl, low mount only, with antenna cable.

- Triangle Reflector Kit

- Fire Extinguisher, Mounted Outboard Driver Seat Hazmat approval UL listed/rated ABC

- Increased Air Filter Full Indication

- Trailer Reservoir Air Pressure

- Main Instrumentation Panel

- Digital Cluster 15" Display with Speedometer, Tachometer, Primary Air Pressure, Secondary Air Pressure, Fuel Level, DEF Level, DPF Filter Status, Fuel Economy, Oil Pressure, Coolant Temp, and Voltmeter.

- Sleeper Digital Alarm Clock

- Two Load Lights, Incandescent, Flush mount Low lnbd Back of cab / back of sleeper

- 44in Sleeper/ Prestige Interior {3M)

** Note here any exceptions taken to specifications/options listed above: