Scope of Work:

The Contractor shall provide all labor, materials, and equipment necessary to repair roof leaks on Building 425 of the Louisiana Naval Air Station for the Department of Military Affairs (DMA).

Location

Naval Air Station

400 Russell Ave.

Belle Chasse, LA 70037

DMA Contact:

Gary Lipkos

(985) 634-5703

Gary.lipkos.1@us.af.mil

Specification:

The Contractor shall be responsible for the following:

  - Supply and install one new metal vent cap to replace the missing cap.

  - Remove and replace existing vent boots with new boots and sealant.

  - Cut out and replace sealant around all vent curb penetrations.

  - Remove and replace existing screws and washers on ridge cap flashing with new screws and washers.

  - Cut out and replace sealant on ridge flashing overlaps, and install new screws.

Terms and Conditions:

  - Upon successful completion of the inspection, the work will be accepted.

  - The Contractor shall secure the building, and any sensitive items at the end of each workday in accordance with the jobsite visit contact.

  - The Contractor shall take all precautions necessary to prevent damages when making repairs. This includes movement and parking of equipment on grass or unimproved areas surrounding site.

  - The Contractor shall be responsible for disposal and storage of all materials being removed or installed throughout the project.

  - The Contractor shall warranty their workmanship and the completed project for a 1 year period. The 1-year period starts with the date of the filing of the certificate of completion.