# GENERAL EQUIPMENT CHARACTERISTICS

Inside Shop with minimal outdoor use

Load Capacity	5000-6000 lbs. (Measured from Load Center, Max. Fork Height)

Drive Power Type			Diesel

Operator Type				Seated

Tire Type				Solid Pneumatic

Standard or dual wheels acceptable.

# GENERAL EQUIPMENT DIMENSIONS

# Must be able to reach 110 in.

# No mast preference.

# Mast Tilting Range 			6o / 6o   (Forward / Backward)

# Forks 					Standard 48-inch forks

Length to Face of Forks		115 in. Maximum

Overall Width				67 in Maximum

Turning Radius 			95 in. Maximum

Seat					Ergonomic; Suspension preferred but not required.

# CHASIS DIMENSIONS

Wheels				2X / 2 (X= Driven Wheels) or 4WD Acceptable

Wheelbase Distance			65 in.

Tires must be appropriately sized for forklift.

# POWERTRAIN

Battery Voltage			12V

Combustion Engine Manufacture	KUBOTA /equal to or better per specifications

Combustion Engine Model		WG2503 /equal to or better per specifications

Rated Output 				61 HP Minimum

Rated Speed				2700rpm Minimum

Number Cylinders/ Displacement	4 Cylinder Minimum

Transmission Type			Powershift

Number of Speeds			1/1 (Forward/Reverse)

# Manufacturer’s Standard Warranty

# ACCESSORY: Automatic Side Shifter/Fork Positioner