# Brand: Tailift or Equal

# Model: PFD30 or Equal

# Please Specify Brand Bidding: ______________________

# Please Specify Model Bidding: __________________________

# GENERAL EQUIPMENT CHARACTERISTICS

Inside Shop with minimal outdoor use

Load Capacity	6000-7000 lbs. (Measured from Load Center, Max. Fork Height)

Drive Power Type			Diesel

Operator Type				Seated

Tire Type				Solid Pneumatic

Specify Load Capacity in Pounds Bidding: ________

Specify Bidding Diesel Drive Power Type: Yes: ______ No:______

Specify Operator Type Bidding: ___________

Specify Tire Type Bidding: ____________

# GENERAL EQUIPMENT DIMENSIONS

# Must be able to reach 110 in.

# Mast Tilting Range 			6o / 6o   (Forward / Backward)

# Forks 					Standard 48-inch forks

Length to Face of Forks		115 in. Maximum

Overall Width 			            56 in Maximum

Turning Radius 			96 in. Maximum

Seat					Standard

Specify Reachable Height Bidding: _________

Specify Mast Tilting Range Bidding: __________

Specify Forks Bidding (inches): ________

Specify Length to Face of Forks Bidding: __________

Specify Overall Width Bidding: ______

Specify Turning Radius Bidding:  ____________

Specify Seat Type Bidding: _____________

# CHASIS DIMENSIONS

Wheels				2X / 2 (X= Driven Wheels) or 4WD Acceptable

Wheelbase Distance			65 in. Maximum

Tires must be appropriately sized for forklift.

Specify 2X/2 or 4WD Bidding: _____________

Specify Wheelbase Distance Bidding: __________

# POWERTRAIN

Battery Voltage			12V

Combustion Engine Manufacture	KUBOTA /equal to or better per specifications

Combustion Engine Model		V2403 /equal to or better per specifications

Rated Output 				57 HP Minimum

Rated Speed				2400rpm Minimum

Number Cylinders/ Displacement	4 Cylinder Minimum

Transmission Type			Powershift

Number of Speeds			1/1 (Forward/Reverse)

Specify Battery Voltage Bidding: ____________

Specify Engine Manufacture Bidding: _______________

Specify Engine Model Bidding: _____________

Specify Rated Output Bidding: ___________

Specify Rated Speed Bidding: __________

Specify Number Cylinders/ Displacement (Minimum) Bidding: ___________

Specify Transmission Type Bidding: ___________

Specify Number of Speeds Bidding: __________

# ACCESSORY

# Automatic Side Shifter/Fork Positioner

# Manufacturer’s Standard Warranty

# Specify Bidding Automatic Side Shifter/Fork Positioner: Yes:_______ No:______

# Specify Warranty Bidding: _______________________

# Specify Delivery Days ARO Bidding: __________