Scope of Work:

The Contractor shall furnish all labor, materials, tools, and equipment necessary to remove the existing metal roof system and install a metal roofing system for the Eastern LA Mental Health System (ELMHS).

Location:

Eastern Mental Health System Carpenter/Sheet Metal Building

4502 Hwy 951

Jackson, La 70748

Agency Contact

Jonathan Holley or Dylan Fury

(225) 634-0202

Specifications:

The Contractor shall be responsible for the following:

- Demolition and Removal

  - Remove all existing metal roof panels from the structure.

  - Haul off and properly dispose of all removed materials in accordance with local regulations.

- Roof Preparation and Repairs

  - Fabricate and install 24-gauge Galvalume metal T-Panel style system

  - Panels installed in accordance with manufacturer specifications and industry best practices.

- Trim, Flashings & Accessories

  - Provide and install drip edge.

  - Install metal closures as required.

  - Install ridge cap.

  - Install gable trim flashing.

  - Install pipe jacks for all roof penetrations.

  - Eliminate existing skylights and install new metal panels in their place.

  - Fabricate and install J-channel flashing on window headers.

  - Provide all required fasteners and sealants for a complete weather-tight system.

General Terms/Conditions:

- The work area shall be kept clean and orderly throughout the project by the Contractor.

- All debris, fasteners, and scrap material shall be removed from the site upon completion by the Contractor.

- The work shall commence upon receipt of notice to proceed and completion of material procurement. On site workdays are Monday – Friday, 7am to 3:30pm.

- All materials shall be new, of commercial quality, and installed per manufacturer’s specifications.

- The work shall be performed by qualified roofing technicians experienced in metal roof installation.

- Sealants shall be high-performance, weather-resistant types compatible with Galvalume materials.

- The following items are not included unless otherwise stated or agreed upon:

  - Structural repairs or replacement of deteriorated decking or framing.

  - Electrical, mechanical, or plumbing modifications.

  - Interior repairs resulting from pre-existing leaks or conditions.

  - Permits or engineering fees unless specified.