**If bidding other than specified, please provide manufacturer’s specs for “or equal” items. **

Scope of Work:

The Contractor shall provide all labor, material, and equipment needed to demolish the current grain bin and construct a grain bin for the Louisiana Department of Corrections (DOC).

Location:

Prison Enterprise – LSP Row Crops

17544 Tunica Trace Hwy 66

Angola, LA 70712

Jobsite Visit Contact:

Allen Barton

225-330-9838

Specifications:

Brand/Model: GSI 4004 Series Grain Bin or Equal

Specify Brand/Model Bidding: _____________

- Diameter: 27 ft.

- Sidewall Rings: 6 rings

- Wall Sheet Corrugation: 4.00 in.

- Height to Eave: 22 ft. 1.88 in.

- Height to Peak: 29 ft. 10.65 in.

- Commodity: Corn

- Commodity Density: 56 lb./bu

- Designed for Free Flowing: 52 lb./cu ft.

- Capacity (values based on 6% compaction):

- Level:

  - -10173 bushels

  - -11939 cu. ft.

  - -338 cubic meters

  - -285 U.S. tons @ 45 lb./cu. ft.

  - -258 M.T.@ 721 kg./cu. m.

- Peaked:

  - -11322 bushels

  - -13287 cu. ft.

  - -376 cubic meters

  - -317 U.S. tons @ 45 lb./cu. ft.

  - -288 M.T. @ 721 kg/cu. m.

- Roof Peak Load Capacity: 6000 lb.

- Ground Snow Load Rating: 30 psf'

- Design Wind Velocity: 105 mph (per ASCE 7-10)

- Seismic: 0 - Not available for this tank

- Galvanization: G90 (Std.)

- Bin Door: 2 ring doors in 1st/2nd ring

- Roof Ring(s): Standard

- Roof Cap Style: Std. Dome Cap-No Flat Top

- Power: Three Phase 230 Volts

Additional Requirements:

- Dryer Fans

- Ceiling Spreader

- Unloading Auger

- Stiffeners

Terms and Conditions:

- The Contractor representatives are subject to a search of their vehicles, possessions and persons. This is necessary to preclude the introduction of weapons, ammunition, explosives, cell phones, alcohol, escape devices, drugs and drug paraphernalia or other forbidden items or contraband into the prison environment. All searches of visitors will be conducted in accordance with Department Regulation No. C-02-005 and Penitentiary Directive No. 09.004 and 09.004A.

- The Contractor representatives are subject to a background check.

- The Contractor representatives shall follow all institutional policies and procedures while performing work on facility grounds.

- The Contractor is subject to a conference/training with facility staff after the contract is executed and before performing any work.