Scope of Work:

The Contractor shall furnish all labor, materials, tools, and equipment necessary to remove and replace the existing metal roofing system on the Main Warehouse for the Eastern LA Mental Health System (ELMHS).

Location:

Eastern Mental Health System Main Warehouse

4502 Hwy 951

Jackson, La 70748

Agency Contact

Jonathan Holley/Dylan Fury

225-634-0202

Specifications:

The Contractor Shall be responsible for the following:

- Demolition and Removal

  - Remove all existing metal roof panels from the designated roof areas.

  - Cut, detach, and remove all roof fasteners, flashings, and accessories.

  - Haul off and dispose of all roofing debris and scrap materials in accordance with local regulations.

- Roof Preparation and Repairs

- Cut out all existing purlins on the front-loading dock side of the building.

- Construct and install new Z-purlins as required to ensure proper alignment and support for the new roofing system.

- Demolish existing roof framing on the lean-to structure, haul off debris, and install new wood framing to accommodate the new roof installation.

- T-Panel Metal Roofing System:

- Construct and install a 24-gauge Galvalume T-Panel metal roofing system with double lock seams.

- Includes installation of:

  - Roof panels (T-Panel system)

  - Drip edge and eave trim

  - Ridge caps and closures

  - Box ridge vents

  - Gable trim flashing

  - Pipe jacks and sealants

  - Fasteners, clips, and all necessary accessories

- Patch and install up to three rolls of new insulation as needed.

- Construct and install T-panel roof panels on both main roof and lean-to framing.

- Ensure all penetrations are properly sealed and waterproof.

- Commercial Gutter System:

  - Fabricate and install new 24-gauge Galvalume commercial-grade gutters compatible with roof system.

  - Includes downspouts, straps, and end caps.

General Terms/Conditions:

- Work area shall be kept clean and orderly throughout the project by the Contractor.

- All debris, fasteners, and scrap material shall be removed from the site upon completion by the Contractor.

- Work shall commence upon receipt of notice to proceed and completion of material procurement. On site workdays are Monday – Friday, 7am to 3:30pm.

- All materials shall be new, of commercial quality, and installed per manufacturer’s specifications.

- Work shall be performed by qualified roofing technicians experienced in metal roof installation.

- Sealants shall be high-performance, weather-resistant types compatible with Galvalume materials.