**If bidding other than specified, please provide manufacturer’s specs for “or equal” items. **

Scope of Work

The Contractor shall supply all labor, rentals, permits and materials to remove and install fuel tanks and pumps for the Department of Corrections (DOC).

Location:

Raymond Laborde Correctional Center

1630 Prison Rd.

Cottonport, LA 71327

Site Visit Contact:

James Desjardine

Cell# 318-305-8751

Office# 318-876-2891, ext #245

Specifications:

The Contractor shall be responsible for the following:

- Supply and install one 3,000-gallon 64" Flame Shield AST or Equal

Specify Brand/Model Bidding: _______________________

- Move and repair usable one 10,000-gallon 96" Fireguard AST provided by the Agency on site.

- Pour one 24' x 35' 4000 psi slab with bollards for tank installation

- Supply and install one 2-product remote fill bow with all necessary piping

- Supply and install three Franklin Electric Petro 2HP or Equal submergible motors with leak detectors

Specify Brand/Model Bidding: _______________________

- Supply and install 84' of 2" black iron product piping w/ all necessary solenoid

valve, ball valves, pressure relief valves, etc.

- Supply and install three PCI pre-plumbed under dispenser containment sumps and one

PCI Transition Sump w/ all necessary shear valves and/or ball valves.

- Supply and install 270' of 2" LCX3000 or Equal Double-wall Fiberglass piping.

Specify Brand/Model Bidding: _______________________

- Supply and install all top of tank hardware including 3" Black iron fill piping, AST Overfill Drop Tube.

- Supply and install three Wayne Select or Equal Product Remote dispensers with all necessary handing hardware.

Specify Brand/Model Bidding: _______________________

- Supply and install one DFS Pro Gauge ATG or Equal automatic tank gauge system w/display printer, two inventory only tank probes, four sensors, and two interstice sensors.

Specify Brand/Model Bidding: _______________________

General Terms:

- The Contractor shall be responsible for delivery and pickup of materials/supplies needed to complete project.

- Disposal of all materials and debris shall be the responsibility of the contractor.

- The Contractor shall be responsible for providing any lifting capability to complete all new installations and repairs.

- The Contractor shall take all precautions necessary to prevent damages when making repairs. This includes movement and parking of equipment on grass or unimproved areas surrounding site.

- The Contractor shall contact, prior to mobilization and demobilization, the Project Manager for staging and for satisfactory completion.