ATTACHMENT B

RFx number: 3000025732	Contract Title: Environmental Testing Services – DPS/LOSCO

The Louisiana Oil Spill Coordinator’s Office (LOSCO) requires laboratory services for the analysis of samples related to oil spill response and Natural Resource Damage Assessment (NRDA) cases. The Vendor shall furnish all personnel, equipment, and materials necessary to analyze the samples and provide appropriate outputs in accordance with the specifications outlined in this Attachment B and Attachment C – Price Sheet.

  - Work Statements

  - Project Details

LOSCO will acquire the samples for analysis and, unless LOSCO and the Vendor mutually agree to another method of transfer, LOSCO will provide them to the Vendor by shipment. The Vendor must be able to receive large coolers from a parcel courier. Upon receipt of the samples, the Vendor will be required to maintain chain of custody records and store the samples at appropriate temperatures from the time of receipt through the time of analysis. Upon completion of analysis, the Vendor will preserve and/or dispose of any remains of the sample and/or extracts created during or remaining after analytical testing in accordance with instructions from LOSCO, as well as any pertinent legal requirements. Additionally, LOSCO may require the storage of samples for prolonged periods of time. Examples of circumstances in which storage would be required include, but are not limited to, a sample requiring storage by a lab until such time that an analysis is required or a second, identical sample requires storage in the event that it is needed for a confirming analysis. Storage requirements are:

| Matrix/Analysis | Storage for Samples | Holding Time to Extraction | Holding Time to Analysis |
| --- | --- | --- | --- |
| VOC Analyses |  |  |  |
| Water | Refrigeration 4ºC ±2º with no headspace; Optional: Preserved with HCl in the field in VOA vial. | Not applicable | 7 days if not acid preserved; 14 days if acid preserved |
| Sediment | Refrigeration 4ºC ±2º For preservation requirements, see SW-846 Method 5035A. | Not applicable | 14 days |
| Oil | Above freezing to 30ºC | Not applicable | No holding time |
| Oily Debris | Refrigeration <6ºC | Not applicable | No holding time |

| PAH, SHC/TEH, Biomarker Analyses |  |  |  |
| --- | --- | --- | --- |
| Water | Refrigeration 4ºC ±2º; Optional: Preserved with 1:1 HCl to pH<2 | 7 days if not acid preserved; 14 days if acid preserved | 40 days from extraction1; except biomarkers no holding time |
| Sediment/Soil (also total solids, grain size and TOC) | Frozen (-20ºC ±10ºC), except Grain Size should not be frozen – store at 4ºC ±2º | 2 Years, except not applicable for Grain Size, Total Solids, and TOC | 40 days from extraction1; except biomarkers grain size, total solids and TOC no holding time. |
| Oil | Above freezing to 30ºC | No holding time | 40 days from extraction1; except biomarkers no holding time |
| Oily Debris | Refrigeration <6ºC | No holding time | 40 days from extraction1; except biomarkers no holding time |

  - Procedure

At the time of sample shipment/transfer, LOSCO will provide written instructions to the Vendor itemizing each analysis that should be performed on each sample. The Vendor will confirm receipt of samples and provide an estimated analysis timeline. At this point, the Vendor may begin work on the tasks detailed by LOSCO. The Vendor must notify LOSCO if any problems arise during the course of this work.

  - Technical Specifications

Laboratory Requirements - The Vendor shall meet or exceed the below specifications related to the analyses listed in Attachment C – Price Sheet and maintain these specifications throughout the duration of the contract.

| Polycyclic Aromatic Hydrocarbons (PAH) Testing Requirements |  |
| --- | --- |
| Reported Analyte |  |
| Naphthalene | C4-Naphthobenzothiophenes |
| C1-Naphthalenes | Benz(a)anthracene |
| C2-Naphthalenes | (a) Chrysene1 |
| C3-Naphthalenes | (b) Chrysene + Triphenylene1 |
| C4-Naphthalenes | C1-Chrysenes |
| Biphenyl | C2-Chrysenes |
| Dibenzofuran | C3-Chrysenes |
| Acenaphthylene | C4-Chrysenes |
| Acenaphthene | (a) Benzo(b)fluoranthene1 |
| Fluorene | (b) Benzo(b + j)fluoranthene1 |
| C1-Fluorenes | (a) Benzo(j + k)fluoranthene1 |
| C2-Fluorenes | (b) Benzo(k)fluoranthene1 |
| C3-Fluorenes | Benzo(a)fluoranthene |
| Anthracene | Benzo(e)pyrene |
| Phenanthrene | Benzo(a)pyrene |
| C1-Phenanthrenes/Anthracenes | Indeno(1,2,3-cd)pyrene |
| C2-Phenanthrenes/Anthracenes | (a) Dibenz(a,h)anthracene1 |
| C3-Phenanthrenes/Anthracenes | (b) Dibenz(a,h + a,c)anthracene1 |
| C4-Phenanthrenes/Anthracenes | Benzo(g,h,i)perylene |
| Dibenzothiophene | Perylene |
| C1-Dibenzothiophenes | Benzo(b)thiophene |
| C2-Dibenzothiophenes | 1,2-Dimethylnaphthalene |
| C3-Dibenzothiophenes | 2,6-Dimethylnaphthalene |
| C4-Dibenzothiophenes | 3,6-Dimethylphenanthrene |
| Benzo(b)fluorene | 1-Ethylnaphthalene |
| Fluoranthene | 9-Methylanthracene |
| Pyrene | 4-Methylbiphenyl |
| C1-Fluoranthenes/Pyrenes | 2-Methylfluoranthene |
| C2-Fluoranthenes/Pyrenes | 1-Methylfluorene |
| C3-Fluoranthenes/Pyrenes | 1-Methylnaphthalene |
| C4-Fluoranthenes/Pyrenes | 2-Methylnaphthalene |
| Naphthobenzothiophene | 1-Methylphenanthrene |
| C1-Naphthobenzothiophenes | 2-Methylphenanthrene |
| C2-Naphthobenzothiophenes | 2,3,5-Trimethylnaphthalene |
| C3-Naphthobenzothiophenes |

|  | Target Method Detection Limit Range |
| --- | --- |
| Sediment/Soil = | 0.1 – 0.5 ng/g dry weight |
| Tissue = | 0.2 – 1.0 ng/g wet weight |
| Water = | 1 – 5 ng/L |
|  | Target Reporting Limit |
| Oil = | 2.0 mg/kg |

1 Analytes listed as "a" and "b" pairs are subject to co-elution, and while reported differently by different analytical laboratories, are typically synonymous. Only one concentration from the "a" and "b" pairs should be reported for any individual sample.

Standard Volatile Organic Compound (VOC) Testing Requirements

Analyte

1,2,4-Trimethylbenzene

1,3,5-Trimethylbenzene

4-Isopropyltoluene

Benzene

Ethylbenzene

Isopropylbenzene

m,p-Xylenes

Naphthalene1

n-Butylbenzene

n-Propylbenzene

o-Xylene

sec-Butylbenzene

Styrene

tert-Butylbenzene

Toluene

| Target Method Detection Limit Range |  |
| --- | --- |
| Sediment/Soil = | 0.1 – 1 ng/g |
| Water = | 0.05 – 0.5 µg/L |
| Target Reporting Limit |  |
| Oil = | 2 mg/kg |

1 Naphthalene is also included in the target analyte list of PAH compounds. The PAH analysis is the preferred method, rather than this volatile method. Thus, if a sample location is analyzed for both PAH and VOC the result from the PAH analysis will be noted in the database as the preferred result

| Tot.Ext.HC+Tot.Res.HC+UCM (C9-C44), Aliphatic+Isoprenoids Requirements |  |  |  |
| --- | --- | --- | --- |
| Analyte | Abbr. | Analyte | Abbr. |
| n-Nonane | nC9 | n-Tricosane | nC23 |
| n-Decane | nC10 | n-Tetracosane | nC24 |
| n-Undecane | nC11 | n-Pentacosane | nC25 |
| n-Dodecane | nC12 | n-Hexacosane | nC26 |
| n-Tridecane | nC13 | n-Heptacosane | nC27 |
| 2,6,10 Trimethyldodecane | 1380 | n-Octacosane | nC28 |
| n-Tetradecane | nC14 | n-Nonacosane | nC29 |
| 2,6,10 Trimethyltridecane | 1470 | n-Triacontane | nC30 |
| n-Pentadecane | nC15 | n-Hentriacontane | nC31 |
| n-Hexadecane | nC16 | n-Dotriacontane | nC32 |
| Norpristane | nPr | n-Tritriacontane | nC33 |
| n-Heptadecane | nC17 | n-Tetratriacontane | nC34 |
| Pristane | Pr | n-Pentatriacontane | nC35 |
| n-Octadecane | nC18 | n-Hexatriacontane | nC36 |
| Phytane | Ph | n-Heptatriacontane | nC37 |
| n-Nonadecane | nC19 | n-Octatriacontane | nC38 |
| n-Eicosane | nC20 | n-Nonatriacontane | nC39 |
| n-Heneicosane | nC21 | n-Tetracontane | nC40 |
| n-Docosane | nC22 |

| TEH = Total Extractable Hydrocarbons with silica gel "clean-up" |  |  |
| --- | --- | --- |
| Σ(C9-C44) |  |  |
| Integration of the FID signal over the entire hydrocarbon range from n-C9 to n-C44 after silica gel cleanup. |  |  |
| TEM = Total Extractable Matter with no extract "clean-up" |  |  |
| Σ(C9-C44) |  |  |
| Integration of the FID signal over the entire hydrocarbon range from n-C9 to n-C44 no silica gel cleanup. |  |  |
| Target Method Detection Limit |  |  |
| Sediment (Alkanes) = | 0.01 μg/g dry weight |  |
| Sediment (TEH) = | 1 μg/g dry weight |  |
| Water (Alkanes) = | 0.8 µg/L |  |
| Target Reporting Limit |  |  |
| Oil (Alkanes) = | 200 mg/kg |  |
| Oil (TEH) = | 200 mg/kg |  |
| Water (TEH/TEM) = | 200 µg/L |  |

| Petroleum Biomarkers for Quantitative Analysis Requirements |  |
| --- | --- |
| Compound * | Quant Ion m/z |
| C23 Tricyclic Terpane (T4) | 191 |
| C24 Tricyclic Terpane (T5) | 191 |
| C25 Tricyclic Terpane (T6) | 191 |
| C24 Tetracyclic Terpane (T6a) | 191 |
| C26 Tricyclic Terpane-22S (T6b) | 191 |
| C26 Tricyclic Terpane-22R (T6c) | 191 |
| C28 Tricyclic Terpane-22S (T7) | 191 |
| C28 Tricyclic Terpane-22R (T8) | 191 |
| C29 Tricyclic Terpane-22S (T9) | 191 |
| C29 Tricyclic Terpane-22R (T10) | 191 |
| 18a-22,29,30-Trisnorneohopane-Ts (T11) | 191 |
| C30 Tricyclic Terpane-22S (T11a) | 191 |
| C30 Tricyclic Terpane-22R (T11b) | 191 |
| 17a(H)-22,29,30-Trisnorhopane-Tm (T12) | 191 |
| 17a/b,21b/a 28,30-Bisnorhopane (T14a) | 191 |
| 17a(H),21b(H)-25-Norhopane (T14b) | 191 |
| 30-Norhopane (T15) | 191 |
| 18a(H)-30-Norneohopane-C29Ts (T16) | 191 |
| 17a(H)-Diahopane (X) | 191 |
| 30-Normoretane (T17) | 191 |
| 18a(H)&18b(H)-Oleananes (T18) | 191 |
| Hopane (T19) | 191 |
| Moretane (T20) | 191 |
| 30-Homohopane-22S (T21) | 191 |
| 30-Homohopane-22R (T22) | 191 |
| T22a-Gammacerane/C32-diahopane | 191 |
| 30,31-Bishomohopane-22S (T26) | 191 |
| 30,31-Bishomohopane-22R (T27) | 191 |
| 30,31-Trishomohopane-22S (T30) | 191 |
| 30,31-Trishomohopane-22R (T31) | 191 |
| Tetrakishomohopane-22S (T32) | 191 |
| Tetrakishomohopane-22R (T33)e | 191 |
| Pentakishomohopane-22S (T34) | 191 |
| Pentakishomohopane-22R (T35) | 191 |
| 13b(H),17a(H)-20S-Diacholestane (S4) | 217 |
| 13b(H),17a(H)-20R-Diacholestane (S5) | 217 |
| 13b,17a-20S-Methyldiacholestane (S8) | 217 |
| 14a(H),17a(H)-20S-Cholestane/ 13b(H).17a(H)-20S-Ethyldiacholestane (S12) | 217 |
| 14a(H),17a(H)-20R-Cholestane 13b(H),17a(H)-20R-Ethyldiacholestane (S17) | 217 |
| Unknown sterane(S18) | 217 |
| 13a,17b-20S-Ethyldiacholestane (S19) | 217 |
| 14a,17a-20S-Methylcholestane (S20) | 217 |
| 14a,17a-20R-Methylcholestane (S24) | 217 |
| 14a(H),17a(H)-20S-Ethylcholestane (S25) | 217 |
| 14a(H),17a(H)-20R-Ethylcholestane (S28) | 217 |
| 14b(H),17b(H)-20R-Cholestane (S14) | 218 |
| 14b(H),17b(H)-20S-Cholestane (S15) | 218 |
| 14b,17b-20R-Methylcholestane (S22) | 218 |
| 14b,17b-20S-Methylcholestane (S23) | 218 |
| 14b(H),17b(H)-20R-Ethylcholestane (S26) | 218 |
| 14b(H),17b(H)-20S-Ethylcholestane (S27) | 218 |
| C26,20R- +C27,20S- triaromatic steroid | 231 |
| C28,20S-triaromatic steroid | 231 |
| C27,20R-triaromatic steroid | 231 |
| C28,20R-triaromatic steroid | 231 |
| * Peak identification provided in parentheses |  |

| Target Reporting Limit |  |  |
| --- | --- | --- |
| Sediments/Soil = | 2 ug/Kg dry weight |  |
| Waters = | 10 ng/L |  |
| Target Reporting Limit |  |  |
| Oil = | 2 mg/Kg |  |

  - Deliverables

The Vendor shall provide analyses as electronic data deliverables (EDDs) in addition to regular printouts, which must be in PDF format. The Vendor must fully document all analyses to allow for independent data validation. The Vendor must work cooperatively with data validators to address and resolve any data validation questions or issues.

4.0     Costs for Services

The Vendor must provide the unit cost for all items on Attachment C – Price Sheet. LOSCO will pay all costs necessary to ship/transfer samples to the Vendor. Therefore, the Vendor should not include shipping/transfer charges in the rates provided in Attachment C – Price Sheet. Rates shall include all direct costs (labor, supplies, equipment, sampling supplies), sample storage and disposal, extraction/preparation, cleanup costs, incidentals and expendables, duplication/copying, communications, taxes, etc.), all indirect costs (fringe, overhead, general and administrative costs), travel expenses associated with each line, and profit. To complete the Attachment C – Price Sheet, bidders are to fill out the columns titled “Matrix” and “Price.” In the “Matrix” column, the bidder shall indicate what substance is being analyzed, such as tissue, sediment, or liquid; multiple matrices for the same analysis can be listed by separating with a slash (Sediment/tissue). In the price column, the bidder shall indicate the price for the given analysis. If there are multiple matrices for a specific analysis the bidder may fill out the price column in two ways: if the price is the same for all matrices, the bidder shall input the appropriate price; if the price differs between matrices, the prices shall be listed in the same order as the matrices, separated by a slash. Bidders are to use the “notes/column” column to indicate any additional detail or pricing information. For instance if an analysis has options for different levels of detail which will affect the price, that information should be listed in this column.

5.0	Confidentiality

All efforts and results of the Vendor activities may be used by the State of Louisiana in litigation and must be held in strict confidence. Information/data in any form (oral, electronic, written, or otherwise) whether provided to the Vendor or generated/obtained by the Vendor in connection with the contract is “Confidential Information,” specifically work product prepared for pending or anticipated litigation. This Confidential Information will be held in strict confidence and should be labeled “Confidential Information – Attorney Work Product” by the Vendor. The Vendor shall not disclose Confidential Information to any person or entity without the prior written approval of LOSCO. The obligations of confidentiality survive the termination of the contract for whatever reason.

6.0	Records Retention

The Vendor agrees to retain all notes, records, electronically stored information and communications, and other documents relevant to the contract and the funds expended hereunder until otherwise instructed by LOSCO.