# Attachment B – Specifications

RFx #3000025958

# SPECIFICATIONS – INTERNATIONAL MODEL CV515 SFA (2025) WITH DURALIFT DPM2-40DU, or equal

## 1. GENERAL DESCRIPTION

These specifications describe the minimum requirements for one new 4x4 truck equipped with a bucket lift. This vehicle shall be furnished complete with all necessary operating accessories, equipment, and manuals. The vehicle shall be new, unused, and of the manufacturer’s latest production model meeting or exceeding the requirements listed herein.

A International Model CV151 SFA (2025) with Duralift DPM2-4ODU was used to develop the specifications below and establish equivalency criteria. Equipment of similar style, type, character, quality, features, and purpose conforming to the following detailed requirements/specifications will be considered. For evaluation purposes, bidder's proposing an exception/equivalent option/feature to those specified herein, may be required to provide manufacturer/product information (catalogue sheets, detailed specifications, pictures, etc.). This information will be evaluated against the minimum requirements of this specification. Proposed submittals that are determined not to be equivalent to the established criteria will be rejected.

## 2. VEHICLE CONFIGURATION

Model: International CV515 SFA (CV515)
Axle Configuration: 4x4
Mission: Bucket Lift Application
Wheelbase: 189.00 in.
CA: 107.80 in.
Axle to Frame: 91.00 in.
GVWR: 23,000 lbs (Calculated)
GCWR: 30,000 lbs (Calculated)
Grade Ability: 43.37% / 3.45% @ 55 MPH
Geared Speed: 103.2 MPH

## 3. ENGINE AND TRANSMISSION

Engine: International 6.6L EPA 2024 Diesel, 350 HP @ 2500 RPM, 750 lb-ft Torque @ 1400 RPM.
Transmission: Allison 2750 RDS, 6-Speed Automatic with Double Overdrive, PTO Provision, and Less Retarder.
Includes Park Pawl with 23,500-lb GVWR and 37,500-lb GCVW Max, On/Off Highway use.
Governor Speed: 350 Peak HP (Max)
PTO Control: Customer Intends to Install PTO at Right Side of Transmission.

## 4. AXLES, SUSPENSION, AND BRAKES

Front Axle: Dana Spicer 60-256, Single Reduction, 7,500-lb Capacity, Hub Piloted Wheels.
Rear Axle: Dana Spicer S16-130, Single Reduction, 15,500-lb Capacity, 190 Wheel Ends, Gear Ratio: 4.30.
Suspension, Rear: Single 15,500-lb Capacity, Vari-Rate Springs.
Brakes: Meritor Quadraulic Hydraulic Disc Type, Four 64mm Diameter Pistons, ABS Equipped.
Parking Brake: Bosch DSSA Type, Hydraulic Brake Chassis, Foot Operated in Cab.
Dust Shields: Provided Front and Rear.

## 5. TIRES AND WHEELS

Front Tires: (4) 225/70R19.5 Load Range H HDR+ (Continental), 643 rev/mile, 87 MPH, Drive.
Rear Tires: (4) 225/70R19.5 Load Range H HDR+ (Continental), 643 rev/mile, 87 MPH, Drive.
Front Wheels: Accuride 50180 Disc, 19.5x6.75 Rims, 8-Stud, 275mm BC.
Rear Wheels: Accuride 50180 Dual Disc, 19.5x6.75 Rims, 8-Stud, 275mm BC.

## 6. CAB AND INTERIOR

Cab Type: Conventional Day Cab.
Cab Interior: Trim Classic, Dark Ash Color.
Seats: Driver High Back with Integral Headrest, Recline, Vinyl, Fixed Lumbar. Passenger Two-Man High Back Bench, Fold-Down Armrest, Vinyl.
Windows: Power (2), Left and Right Door.
Air Conditioning: Roof Mounted, Heater, Single Zone.
Mirrors: Dual Heated, Power Adjust, Heated Turn Signal Indicators.
Glass: All Windows Solar Absorbing, Tint.
Air Bags: Front Driver & Passenger, Seat Mounted Side Impact, Side Curtain.

## 7. ELECTRICAL AND LIGHTING

Alternator: Denso SC6, 12V, 220 Amp.
Battery System: (2) VARTA 12V 1300CCA Maintenance-Free.
Lighting: Halogen Composite Headlights, Amber LED Clearance/Marker Lights, Daytime Running Lights.
Trailer Connector: 7-Way Socket Rear of Frame.
Rear View Camera System with Interface to Monitor.
Back-Up Alarm: Electric, 102 dBA.
Radio AM/FM/USB Input/SD Card Slot with 4.2” Color Display.
Speakers: (6).

## 8. BODY AND EQUIPMENT

Bucket Lift: Duralift DPM2-40DU, Telescopic/Articulating Aerial Device.
Body Builder Wiring: Separate Ground/Backup, Stop/Turn/Tail Circuits.
DEF Tank: 6.75 US Gal (26L), Mounted Outside Right Rail.
Fuel Tank: 40 US Gal (151L), Plastic Rectangular, Mounted Between Frame Rails.
Frame: High Strength Low Alloy Steel, 50,000 PSI Yield.
Front Bumper: Contoured, Painted Black.
Paint: Base Coat/Clear Coat, 2-Tone (International White, Code 9219).

Body to include tool compartments.

## 9. OUTRIGGERS

Hydraulic A-frame with integrated sub-base

Located directly behind the cab
Maximum spread: 115 inches
 Electric interlock
 Weather-proof sealed electric control switches, mounted in aluminum boxes at rear

## 10. COMPLIANCE AND WARRANTY

Vehicle shall comply with EPA 2025 emission standards and all applicable FMVSS requirements.
Warranty: Standard CV515 Warranty, Effective with Vehicles Built December 3, 2018 or Later, CTS-3000A.
The vehicle shall include all manufacturer’s standard printed warranties covering defects in materials and workmanship.

# BIDDER’S EXCEPTIONS

Instructions: Bidder should note all exceptions in space provided below. List the detail number from the

aforementioned specification in the column to the left and the exception in the column to the right. Responses may be typed or hand-written. Handwritten responses must be legible. If additional space is needed, please print a duplicate copy of this sheet. "Bidder’s Exceptions" page(s) should be returned with the bid submittal.

Examples:

1.6

Engine has 325 horsepower

1.18.3

Batteries have 2000 CCA combined.

2.2.8

Crossmembers are 4” channel on 12” centers.

Spec. /Detail

Reference	Exception