# Attachment B – Specifications

RFx #3000025959

# SPECIFICATIONS – INTERNATIONAL MODEL CV515 SFA (2026) WITH MORGAN REFRIGERATED BODY & THERMOKING T590-50, or equal;

## 1. GENERAL DESCRIPTION

These specifications describe the minimum requirements for one new 4x2 truck equipped with a 16-foot refrigerated body and reefer unit. The vehicle shall be furnished complete with all necessary operating accessories, equipment, and manuals. The vehicle shall be new, unused, and of the manufacturer’s current production model meeting or exceeding the specifications listed below.

A International Model CV515 SFA (2026)with Morgan refrigerated body and Thermoking T590-50 was used to develop the specifications below and establish equivalency criteria. Equipment of similar style, type, character, quality, features, and purpose conforming to the following detailed requirements/specifications will be considered. For evaluation purposes, bidder's proposing an exception/equivalent option/feature to those specified herein, may be required to provide manufacturer/product information (catalogue sheets, detailed specifications, pictures, etc.). This information will be evaluated against the minimum requirements of this specification. Proposed submittals that are determined not to be equivalent to the established criteria will be rejected.

## 2. VEHICLE CONFIGURATION

Model: International CV515 SFA (CV515)
Axle Configuration: 4x2
Mission: Refrigerated Van
Wheelbase: 235.00 in.
CA: 119.80 in.
Axle to Frame: 75.00 in.
GVWR: 19,500 lbs (Calculated)
GCWR: 26,000 lbs (Calculated)
Grade Ability: 56.69% / 5.05% @ 55 MPH
Geared Speed: 102.5 MPH

## 3. ENGINE AND TRANSMISSION

Engine: International 6.6L EPA 2024 Diesel, 350 HP @ 2500 RPM, 750 lb-ft Torque @ 1400 RPM, 2900 RPM governed speed.
Transmission: Allison 1700 HS, 6-Speed Automatic, Double Overdrive, PTO Provision, Less Retarder, Includes Park Pawl.
Governor Speed: 350 Peak HP (Max)

Exhaust Brake: Engine Mounted for International 6.6L Engine.
Manual Regen Capability Included.

## 4. AXLES, SUSPENSION, AND BRAKES

Front Axle: Dana Spicer D-800N I-Beam Type, 6,250-lb Capacity.
Rear Axle: Dana Spicer S14-110 Single Reduction, 13,500-lb Capacity, 190 Wheel Ends, Gear Ratio: 4.30.
Suspension: Front Parabolic Taper Leaf, 6,250-lb Capacity, with Shock Absorbers. Rear Single 13,500-lb Capacity, Vari-Rate Springs.
Brakes: Meritor Quadraulic Hydraulic Disc Type with ABS, Four Channel Stability Control.
Parking Brake: Bosch DSSA Type, Hydraulic, Foot Operated in Cab.
Dust Shields: Provided Front and Rear.

## 5. TIRES AND WHEELS

Front Tires: (2) 225/70R19.5 Load Range G HSR 5 (Continental), 647 rev/mile, 87 MPH, All-Position.
Rear Tires: (4) 225/70R19.5 Load Range G HDR 5 (Continental), 647 rev/mile, 87 MPH, Drive.
Front Wheels: Accuride 50180, 19.5x6.75 Rims, Powder Coat Steel, 8-Stud, 275mm BC.
Rear Wheels: Accuride 50180 Dual Disc, 19.5x6.75 Rims, Powder Coat Steel, 8-Stud, 275mm BC.

## 6. CAB AND INTERIOR

Cab Type: Conventional 6-Man Crew Cab.
Cab Interior: Trim Classic, Dark Ash Color.
Seats: Driver High Back Vinyl Reclining Seat with Lumbar. Front Passenger Bench with Vinyl Cover. Rear Bench Cloth Seat.
Mirrors: Dual Heated Power Mirrors, Manual Folding & Extending.
Windows: Power (4), Front and Rear Doors.
Air Conditioning: Roof Mounted, Heater, Single Zone.
Glass: Solar Absorbing, Tinted.
Air Bags: Dual Front, Side, and Curtain Airbags with Outboard Seat-Mounted Protection.

## 7. ELECTRICAL AND LIGHTING

Alternator: Denso SC6, 12V, 220 Amp.
Battery System: (2) VARTA 12V, 1100CCA, Maintenance-Free.
Lighting: Halogen Composite Aero Headlights, Amber LED Clearance/Marker Lights, Daytime Running Lights.
Trailer Connector: 7-Way Socket Rear of Frame.
Rear Camera System: Integrated with Color Display.
Back-Up Alarm: Electric, 102 dBA.
Radio: AM/FM/USB/SD/AUX with 4.2” Touch Screen and Six Speakers.

## 8. BODY AND REFRIGERATION EQUIPMENT

Body: Morgan 16-ft Refrigerated Van Body with Theiman TV20 Lift Gate.
Refrigeration Unit: ThermoKing T590-50 Reefer Unit.
Fuel Tank: 40 US Gal (151L) Plastic Rectangular, Mounted Between Frame Rails.
DEF Tank: 6.75 US Gal (26L), Frame Mounted Outside Right Rail, Under Cab.
Frame Rails: High Strength Low Alloy Steel, 50,000 PSI Yield.
Paint: Base Coat/Clear Coat, Winter White (Location Code 9219).

## 9. COMPLIANCE AND WARRANTY

Vehicle shall comply with EPA 2025 emission standards and FMVSS requirements.
Warranty: Standard CV515 Warranty, Effective for Vehicles Built December 3, 2018 or Later, CTS-3000A.
Includes manufacturer’s standard printed warranties covering defects in materials and workmanship.

# BIDDER’S EXCEPTIONS

Instructions: Bidder should note all exceptions in space provided below. List the detail number from the

aforementioned specification in the column to the left and the exception in the column to the right. Responses may be typed or hand-written. Handwritten responses must be legible. If additional space is needed, please print a duplicate copy of this sheet. "Bidder’s Exceptions" page(s) should be returned with the bid submittal.

Examples:

1.6

Engine has 325 horsepower

1.18.3

Batteries have 2000 CCA combined.

2.2.8

Crossmembers are 4” channel on 12” centers.

Spec. /Detail

Reference	Exception