# ATTACHMENT B

## RFx Number: 3000025941       Contract Title: Emergency Support Trailers - SW

Specifications

Specifications for Emergency Mobile Trailers: Showers, Restrooms, Laundries and Bunkhouses

The State will provide the Contractor with a headcount, and any special needs for the designated deployment site prior to the mobilization of trailers. Contractor will be advised by the State if mobile trailers are required for first responders or special needs shelters that may require Americans with Disabilities Act (ADA) compliant trailers. The Contractor shall provide trailers to accommodate the needs of each location based on information provided by the State.

Contractors are expected to be totally self-sufficient in terms of providing transport, potable water storage, black/grey water storage/removal/disposal/dump fees, electric power, fuel for operations, and solid waste disposal.  No State furnished equipment or support should be assumed.

Contractors shall provide sturdy steps and or handicap ramps that have solid handrails and comply with all current Occupation Safety and Health Act (OSHA) and or ADA standards.

Adequate fire extinguishers shall be provided meeting current OSHA, National Fire Protection Association 10 requirements.

Contractor is to follow all Department of Transportation regulations at all times.

No alcoholic beverages, controlled substances or firearms may be taken to the designated site.

Contractor employees shall wear a badge identifying the name of the employee and the company. Contractor shall ensure that employees are neat and clean in appearance.  Contractor shall provide sleeping provisions and meals for Contractor personnel for the duration of the operation and demobilization.  Contractor to ensure that only those Contractor employees essential to the operation remain at the designed site.

The Contractor shall demobilize functional activity, clean the site, remove and dispose of all garbage/waste, used materials and return the site to its original condition within 12 hours of the demobilization notice.

------------------------------------------------------------------------------------------------------------------

Mobile Showers

Self-Sustaining Mobile Showers Specifications

Upon notification from the State, the Contractor shall provide all physical and ancillary equipment necessary for a completely self-sustaining mobile shower operation to include; potable water, gray water storage, removal and disposal, waste management, power generation, and propane.

The Contractor shall ensure mobile showers comply with OSHA CFR 29 1910.142, Temporary Labor Camps, Uniform Plumbing Code, National Sanitation Foundation Standards, and ANSI Z4.4, Sanitation in Field and Temporary Labor Camps.  Shower facilities shall have nonskid floors, individual stalls with privacy curtains, sinks with mirrors, outlets for hairdryers, soap/shampoo caddies, and adequate dressing areas.  Units shall have hot and cold water serviced by an ASME rated propane water heater and an anti-scald valve.  Hot water shall be a minimum of 110 º F.  Contractor is responsible for compliance with all local, state, and federal regulations and laws, including all Louisiana Department of Health (LDH) regulations including all permits related to Contractor operations.

The Contractor should furnish the following:

  - All labor to include; but not limited to, complete management, control, transport, purchase, receipt, storage, issue, set up, handling, processing, drying towels (paper),  phosphate free soap for dispensers, and shaving equipment.

  - Garbage cans and liners for outside shower unit.

  - Temporary waiting area with chairs and grass rugs sufficient to stage 10 personnel.

  - A 5 gallon water bucket with chlorine solution for flip-flops used for showering.

The Contractor should ensure the following:

  - Certificated potable water: initial supply of 1500 gallons of potable water with recurring certificated potable water reorders using a 6,000-8,000 gallon tanker.  Provided equipment shall be in accordance with all current Federal and State potable water codes; the current Food Code issued by the U.S. Department of Health and Human Services; U.S. Public Health Service, Food and Drug Administration.

  - Gray water storage capacity of a minimum of two 1350 gallon bladder bags per shower.

  - The shower unit shall have the ability to be segregated by male and female and be fully enclosed except the shower access door.

  - All pipes, fittings, or hoses should comply with the appropriate codes.  All equipment should preserve the potable water quality throughout the shower unit and peripheral equipment where water is stored and used. Space around pipes, conduits, or hoses that extend through floors or outer walls should be sealed.  The seal should be smooth and easily cleanable.

  - Light bulbs, tubes, etc., should be covered with completely enclosed plastic safety shields, approved shatterproof type bulbs or the equivalent.  Light fixtures should be installed so as to not constitute a hazard to personnel.  No electrical receptacles will be placed inside the showers.

Sanitation Requirements

The inside of the mobile shower shall be sanitized in its entirety no less than twice per day.  The interior shall be washed down with soap or detergent, rinsed thoroughly, sanitized with a household bleach solution using a minimum of 1 tablespoon of bleach per 2 gallons of water, then rinsed thoroughly.  Floor mats shall be removed and sanitized twice daily.  Shower curtains shall be sanitized twice daily and replaced as needed.

Self-Sustaining ADA Compliant Mobile Showers Specification

Upon notification from the State, the Contractor shall provide all physical and ancillary equipment necessary for a completely self-sustaining ADA compliant mobile shower operation to include; potable water, gray water removal, waste management, power generation, and propane.

The Contractor shall ensure mobile showers comply with OSHA CFR 29 1910.142, Temporary Labor Camps, Uniform Plumbing Code, National Sanitation Foundation Standards, and ANSI Z4.4, Sanitation in Field and Temporary Labor Camps.  Shower facilities shall have nonskid floors, individual stalls with privacy curtains, sinks with mirrors, outlets for hairdryers, soap/shampoo caddies, and adequate dressing areas.  Units shall have hot and cold water serviced by an ASME rated propane water heater and an anti-scald valve.  Hot water shall be a minimum of 110 º F.  Contractor is responsible for compliance with all local, state, and federal regulations and laws, including all Louisiana Department of Health (LDH) regulations including all permits related to Contractor operations.

The Contractor should furnish the following:

  - All labor to include; but not limited to, complete management, control, transport, purchase, receipt, storage, issue, set up, handling, processing, drying towels (paper), phosphate free soap for dispensers, and shaving equipment.

  - Garbage cans and liners for outside shower unit.

  - Temporary waiting area with chairs and grass rugs sufficient to stage 10 personnel.

  - A 5 gallon water bucket with chlorine solution for flip-flops used for showering.

The Contractor should ensure the following:

- Certificated potable water: initial supply of 1500 gallons of potable water with recurring certificated potable water reorders using a 6,000-8,000 gallon tanker.  Provided equipment shall be in accordance with all current Federal and State potable water codes; the current Food Code issued by the U.S. Department of Health and Human Services; U.S. Public Health Service, Food and Drug Administration.

- Gray water storage capacity of a minimum of two 1350 gallon bladder bags per shower.

- The shower unit shall have the ability to be segregated by male and female and be fully enclosed except the shower access door.

- All pipes, fittings, or hoses should comply with the appropriate codes.  Space around pipes, conduits, or hoses that extend through floors or outer walls should be sealed.  The seal should be smooth and easily cleanable.

- Light bulbs, tubes, etc., should be covered with completely enclosed plastic safety shields, approved shatterproof type bulbs or the equivalent.  Light fixtures should be installed so as to not constitute a hazard to personnel.  No electrical receptacles will be placed inside the showers.

Sanitation Requirements

The inside of the mobile shower shall be sanitized in its entirety no less than twice per day. The interior shall be washed down with soap or detergent, rinsed thoroughly, sanitized with a household bleach solution using a minimum of 1 tablespoon of bleach per 2 gallons of water, then rinsed thoroughly.  Floor mats shall be removed and sanitized twice daily.  Shower curtains shall be sanitized twice daily and replaced as needed.

Mobile Restroom

Self-Sustaining Mobile Restroom Trailer Specifications

  - Upon notification from the State, the Contractor shall provide all physical and ancillary equipment necessary for a completely self-sustaining mobile restroom operation to include; potable water, black/gray water removal, waste management, power generation, propane.

  - Self-contained for 100% of the NIMS (ratio of at least 1 toilet to 20 persons, or 1 toilet and 1 urinal per 25 males) in accordance with ANSI Z4.3 and Z4.4. Sink ratio of 1 per 10 persons in restrooms.

  - Male/Female Segregation – Assume 60% male and 40% female. The ratio may be adjusted by the State as required.

The Contractor should furnish the following:

  - All labor to include; but not limited to, complete management, control, transport, purchase, receipt, storage, issue, set up, handling, processing, drying towels (paper), and phosphate free soap for dispensers.

  - Garbage cans and liners for outside restroom unit.

The Contractor should ensure the following:

  - Certificated potable water: initial supply of 1500 gallons of potable water with recurring certificated potable water reorders using a 6,000-8,000 gallon tanker.  Provided equipment shall be in accordance with all current Federal and State potable water codes; the current Food Code issued by the U.S. Department of Health and Human Services; U. S. Public Health Service, Food and Drug Administration.

  - Black/gray water storage capacity of a minimum of two 1350 gallon bladder bags per toilet.

  - Equipment surface shall be free from channels, crevices, flanges, ledges, sharp or jagged edges, and other cleaning obstructions. Unfinished wooden surfaces are not permitted.

  - All pipes, fittings, or hoses should comply with the appropriate codes.  All equipment should preserve the potable water quality throughout the restroom unit and peripheral equipment where water is stored and used. Space around pipes, conduits, or hoses that extend through floors or outer walls should be sealed.  The seal should be smooth and easily cleanable.

  - Light bulbs, tubes, etc., should be covered with completely enclosed plastic safety shields, approved shatterproof type bulbs or the equivalent.  Light fixtures should be installed so as to not constitute a hazard to personnel.  No electrical receptacles will be placed inside the restrooms.

Sanitation Requirements

The inside of the mobile restroom shall be sanitized no less than twice per day.  The floors shall be washed down with soap or detergent, rinsed thoroughly, sanitized with a household bleach solution using a minimum of 1 tablespoon of bleach per 2 gallons of water, then rinsed thoroughly. Toilets and urinals, wash basin sinks, and toilet seats shall be sanitized with cleaner containing disinfecting bleach.   Restroom partitions and doors shall be spot washed, free of writing, dust and stains.  Mirrors must be clean and free of water spots, and streaks.  All restroom dispensers must be stocked and maintained at adequate levels.

Self-Sustaining ADA Compliant Mobile Restroom Trailer Specifications

  - Upon notification from the State, the Contractor shall provide all physical and ancillary equipment necessary for a completely self-sustaining ADA compliant mobile restroom operation to include; potable water, black/gray water removal, waste management, power generation, and propane.

The Contractor should furnish the following:

  - All labor to include; but not limited to, complete management, control, transport, purchase, receipt, storage, issue, set up, handling, processing, drying towels (paper), and phosphate free soap for dispensers.

  - Garbage cans and liners for outside restroom unit.

The Contractor should ensure the following:

  - Certificated potable water: initial supply of 1500 gallons of potable water with recurring certificated potable water reorders using a 6,000-8,000 gallon tanker.  Provided equipment shall be in accordance with all current Federal and State potable water codes; the current Food Code issued by the U.S. Department of Health and Human Services; U.S. Public Health Service, Food and Drug Administration.

  - Black/gray water storage capacity of a minimum of two 1350 gallon bladder bags per toilet.

  - The restroom unit shall have the ability to be segregated by male and female and be fully enclosed.

  - Equipment surface shall be free from channels, crevices, flanges, ledges, sharp or jagged edges, and other cleaning obstructions. Unfinished wooden surfaces are not permitted.

  - All pipes, fittings, or hoses should comply with the appropriate codes.  All equipment should preserve the potable water quality throughout the restroom unit and peripheral equipment where water is stored and used. Space around pipes, conduits, or hoses that extend through floors or outer walls should be sealed.  The seal should be smooth and easily cleanable.

  - Light bulbs, tubes, etc., should be covered with completely enclosed plastic safety shields, approved shatterproof type bulbs or the equivalent.  Light fixtures should be installed so as to not constitute a hazard to personnel.  No electrical receptacles will be placed inside the restroom.

Sanitation Requirements

The inside of the mobile restroom shall be sanitized no less than twice per day.  The floors shall be washed down with soap or detergent, rinsed thoroughly, sanitized with a household bleach solution using a minimum of 1 tablespoon of bleach per 2 gallons of water, then rinsed thoroughly. Toilets and urinals, wash basin sinks, and toilet seats shall be sanitized with cleaner containing disinfecting bleach.   Restroom partitions and doors shall be spot washed, free of writing, dust and stains.  Mirrors must be clean and free of water spots, and streaks.  All restroom dispensers must be stocked and maintained at adequate levels.

Mobile Laundry Services

Self-Sustaining Laundry Operations Overview

Upon notification, Contractor will provide all physical and ancillary equipment necessary to provide a completely self-sustaining laundry service to include; potable water, gray water removal, waste management, power generation, and propane.   Contractor should provide laundry trailers to accommodate site requirements.

Laundry Specifications

The Contractor should furnish the following:

- All labor to include a fluff & fold service, control, transport, purchase, receipt, storage, issue, set up, handling, processing, laundry soap, bleach and laundry bags.

- Garbage cans and liners for outside the laundry unit.

The Contractor should ensure the following:

- Certificated potable water: initial supply of 1500 gallons of potable water with recurring certificated potable water reorders using a 6,000-8,000 gallon tanker.   Provided equipment shall be in accordance with all current Federal and State potable water codes; the current Food Code issued by the U.S. Department of Health and Human Services; U.S. Public Health Service, Food and Drug Administration.

- Gray water storage capacity of a minimum of one 1350 gallon bladder bag per washer.

- Provide equipment and operate in accordance with all current Federal, State, local laws or standards; OSHA regulations; the National Electrical Code (NEC); the Uniform Plumbing Code (UPC); Federal and State potable water codes; the current Food Code issued by the U.S. Department of Health and Human Services; U.S. Public Health Service, Food and Drug Administration; and other contractual requirements stated herein.

- All pipes, fittings, or hoses should comply with the appropriate codes.  All equipment should preserve the potable water quality throughout the laundry unit and peripheral equipment where water is stored and used. Space around pipes, conduits, or hoses that extend through floors or outer walls should be sealed.  The seal should be smooth and easily cleanable.

- Light bulbs, tubes, etc., should be covered with completely enclosed plastic safety shields, approved shatterproof type bulbs or the equivalent.  Light fixtures should be installed to not constitute a hazard to personnel.

Mobile Bunkhouse

Self-Sustaining Mobile Bunkhouses Specification

Upon notification from the State, the Contractor shall provide all physical and ancillary equipment necessary for a completely self-sustaining mobile bunkhouse operation to include; potable water, gray water and black water storage tanks and removal, waste management, power generation, and propane.

The Contractor should furnish the following:

- All labor to include; but not limited to, complete management, control, transport, purchase, receipt, storage, issue, set up, handling, processing, drying hand towels (paper),  drying bath towels (paper) and antibacterial phosphate free soap for dispensers and shower soap.

- All bed linens including pillows, sheets, blanket and mattress.  All bed linens should be changed every 2 days.

- Curtains on all berths for privacy.

- Attached ladder system providing access to upper bunks.

- Garbage cans and liners for outside bunkhouse unit.

- Exterior area lighting.

- Heavy duty entrance doors with stairs or ramps.

The Contractor should ensure the following:

- Certified potable water must be supplied for mobile bunkhouses in quantities capable of providing adequate support based on the number of persons being housed at the site.  Provided equipment shall be in accordance with all current Federal and State potable water codes; the current Food Code issued by the U.S. Department of Health and Human Services; U.S. Public Health Service, Food and Drug Administration.

- Gray water and black water storage capacity must be adequate to support bunkhouses based on the number of persons being housed at the site.

- Gray water and black water removal.

- Hot water heater with an adequate capacity to support bunkhouses based on the number of persons being housed at the site.

- Includes an attendant whose function is to maintain a daily berth rooster and provide a thorough daily cleaning of sleeping areas and sanitizing of restroom areas.

- HVAC system capable of distribution throughout berths, and restrooms areas maintaining adequate air flow to sustain a temperature range of approximately ±72° F.

- Provide equipment and operate in accordance with all current Federal, State, local laws or standards; OSHA regulations; the National Electrical Code (NEC); the Uniform Plumbing Code (UPC); Federal and State potable water codes; the current Food Code issued by the U.S. Department of Health and Human Services; U.S. Public Health Service, Food and Drug Administration; and other contractual requirements stated herein.

- The bunkhouse unit shall have the ability to be segregated by male and female and be fully enclosed except the bunkhouse access door.

- Equipment surface shall be free from channels, crevices, flanges, ledges, sharp or jagged edges, and other cleaning obstructions. Unfinished wooden surfaces are not permitted.

- All pipes, fittings, or hoses should comply with the appropriate codes.  All equipment should preserve the potable water quality throughout the bunkhouse unit and peripheral equipment where water is stored and used. Space around pipes, conduits, or hoses that extend through floors or outer walls should be sealed.  The seal should be smooth and easily cleanable.

- Light bulbs, tubes, etc., should be covered with completely enclosed plastic safety shields, approved shatterproof type bulbs or the equivalent.  Light fixtures should be installed so as to not constitute a hazard to personnel.  No electrical receptacles will be placed inside the bunkhouses.

Contractor Mobilization and Delivery Information (ARO = After Receipt of Order)

Vendor Deployment Location: ____________________

Hours after ARO to departure ____________________

Estimated Transit Times Under Good Conditions

Alexandria, LA: ____________________

Baton Rouge, LA: ____________________

LaPlace, LA: ____________________

Lake Charles, LA: ____________________

Shreveport, LA: ____________________

Vendor Secondary Deployment Location: ____________________

Hours after ARO to departure ____________________

Estimated Transit Times Under Good Conditions

Alexandria, LA: ____________________

Baton Rouge, LA: ____________________

LaPlace, LA: ____________________

Lake Charles, LA: ____________________

Shreveport, LA: ____________________