Scope of Work:

The Contractor shall provide equipment, labor and materials to make improvements to the Zeek All-Terrain Vehicle (ATV) Trail and the Crouch ATV Trail for the Louisiana Department of Wildlife and Fisheries (LDWF).

Location:

Richard K. Yancey Wildlife Management Area

12648 Highway 15

Vidalia, Louisiana 71373

Agency Contact:

Schuyler Dartez; sdartez@wlf.la.gov; (337) 735-8682

Arthur Hebert; ahebert@wlf.la.gov; (337) 735-8681

Specifications:

The Contractor shall be responsible for the following:

Zeek ATV Trail

  - The trail right-of-way shall be 50 feet in width and approximately 2.0 miles in length.

  - Top width shall be of a minimum of 15 feet between side slopes.

  - Top elevation shall be a minimum of 1 foot above grade using hauled in fill material or clean on-site fill material.

  - Side slopes shall be constructed at a maximum 3:1 slope with V-ditches on each side of the trail. Ditch depth shall be a maximum of 1 foot below grade. Lateral drains shall be constructed where needed.

  - Trees, brushes, overhanging brush, and vegetation shall be removed from the trail right-of-way.

  - The trail is 3.0 miles long, but only the first 2 miles shall be included in this project.

  - Add and grade fill material and limestone on the 100 feet x 80 feet parking area at Zeek Trail Parking Area.

Crouch ATV Trail

- The trail right-of-way shall be 50 feet in width and approximately 2.9 miles in length.

- Top width shall be of a minimum of 15 feet between side slopes.

- Top elevation shall be a minimum of 1 foot above grade using hauled in fill material or clean on-site fill material.

- Side slopes shall be constructed at a maximum 3:1 slope with V-ditches on each side of the trail. Ditch depth shall be a maximum of 1 foot below grade. Lateral drains shall be constructed where needed.

- Trees, brushes, overhanging brush, and vegetation shall be removed from the trail right-of-way.

- Add and grade fill material and limestone on the 100 feet x 80 feet parking area at Crouch Trail Parking Area.

The Contractor shall provide and install or replace the following:

Zeek ATV Trail

    - Six 12 gauge, galvanized steel, corrugated, polymer coated culverts shall be 18 inches in diameter and 30 feet in length.

    - One 12 gauge, galvanized steel, corrugated, polymer coated shall be 36 inches in diameter and 40 feet in length.

    - Up to 650 cubic yards of fill material shall be added to elevate the trail. On-site fill material from ditch digging and trail reshaping can be used to elevate the trail. Material used by the Contractor shall be clean of wood and organic debris.

    - 60 tons of #610 limestone shall be added and graded on the 100 feet x80 feet parking area at Zeek Trail Parking Area.

On Crouch ATV Trail

    - Four 12 gauge, galvanized steel, corrugated, polymer coated culverts to be 18 inches in diameter and 30 feet in length.

    - One 12 gauge, galvanized steel, corrugated, polymer coated to be 24 inches in diameter and 30 feet in length.

    - Up to 650 cubic yards of fill material shall be added to elevate the trail. On-site fill material from ditch digging and trail reshaping can be used to elevate the trail. Material used by the Contractor shall be clean of wood and organic debris.

    - 30 tons of #610 limestone shall be added and graded on the 100 feet x 80 feet parking area at Crouch Trail Parking Area.

    - All stumps, limbs, trees, or other woody vegetation removed shall be pushed off of right-of-way or burned.

General Information:

- The Contractor shall be responsible for field verifying all dimensions and existing site conditions of work to be performed.

- The Contractor shall take all precautions necessary to prevent damages when delivering materials to site and provide covering to prevent rusting.