# Attachment E                                                                                                                                                                                         RFx number: 3000026031       Contract Title: Emergency Safety and Traffic Supplies

Barricade Warning Lights

Product ID 10143 – Type A/C Combo Barricade Warning Light

Product ID 10144 – Type B Barricade Warning Light

Barricade warning lights must meet there requirements of the 2026 LA Roads and Bridges Specification Section 1018.13

1018.13 Barricade Warning Lights

1018.13.010 General

Unless otherwise designated in the plans, barricade warning lights shall be Type A/C (switchable combination low-intensity flashing and steady burn), Type B (high-intensity flashing), or Type D (360-degree steady burn), and all bulbs shall be LED-type. Barricade warning lights shall be from the Approved Materials List, and comply with the MUTCD.

These lights are not for warning gates or barriers for moveable bridges.

1018.13.020 Markings

Each light submitted for approval and each light placed on a project shall have a permanently attached identification plate or other permanent markings with the following information:

a) Manufacturer's name

b) Model number

c) Type

d) Lens manufacturer and identification number

e) Circuit manufacturer and identification number

f) Bulb number

g) Minimum operating voltage required to conform to minimum intensity requirements

h) Year of manufacture

1018.13.030 Certification

Prior to installation, furnish the engineer with the following information:

a) Material certification (Certificate of Compliance)

b) Proposed number of warning lights to be used

c) Type

d) Trade name

e) Manufacturer's name and model number

The certification shall also state that each light assembly has been tested, is functioning properly and will be maintained in satisfactory working order.

Barricade warning lights must be an approved product listed on the following Approved Materials List (AML):

Approved Materials List

| Product Category : Barricade Warning Lights |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | MATERIAL : 1018M00070 Barricade Warning Light |  |  |  |
| APS00002090 Dicke Safety Products - Downers Grove, IL 1201 Warren Avenue Downers Grove, IL 60515 Product Approved On Remarks D-Cell Light AC4D-Type A/C - DickeSafety 5-31-2006 D-Cell Light B4DH Type B - Dicke Safety 4-27-2016 Solar/D-Cell-AC4DSL-NL-TypeA/C-DickeSaf 4-27-2016 |  | Phone : 630-207-5224 Ext. # : Fax : 630-969-3973 Contact : Greg Spear (greg@dicketool.com) |  |  |
| APS00002110 Empco-Lite - Elgin, IL |  | Phone : 847-931-2455 Ext. # : Fax : 847-931-2454 Contact : LeRoy Goff (lgoff@empco-lite.com) |  |  |
| Division of Elgin Molded Plastics |  |  |  |  |
| 909 Grace Street |  |  |  |  |
| Elgin, IL 60120 |  |  |  |  |
| Product Approved On Remarks |  |  |  |  |
| Empco-Lite 1102 LED - Type B 6-10-2008 |  |  |  |  |
| Empco-Lite 2006 - Type A/C 6-10-2008 |  |  |  |  |
| Empco-Lite 212 - Type B 6-10-2008 |  |  |  |  |
| Empco-Lite 212-3 - Type B 6-10-2008 |  |  |  |  |
| Empco-Lite 212-3DH - Type B 6-10-2008 |  |  |  |  |
| Empco-Lite 212-3LW - Type B 6-10-2008 |  |  |  |  |
| Empco-Lite 212-3S - Type B 6-10-2008 |  |  |  |  |
| Empco-Lite 212-6 - Type B 6-10-2008 |  |  |  |  |
| Empco-Lite 212-6LW - Type B 6-10-2008 |  |  |  |  |
| Empco-Lite 400 LED - Type A/C 6-10-2008 |  |  |  |  |
| Empco-Lite 499L3 - Type D 6-10-2008 |  |  |  |  |
| Empco-Lite Toughlite 2000 - Type A/C 2-21-2018 |  |  |  |  |
| Empco-Lite Y2K Lightweight - Type A/C 6-10-2008 |  |  |  |  |
| APS00002130 Interplex Solar AER, LLC - East Haven, CT |  | Phone : Fax : Contact : | 800-889-0870 Ext. # : 203-468-6109 John Leopardi (customerservice@interplexsolar.com) |  |
| 192 Commerce St. |  |  |  |  |
| East Haven, CT 06512 |  |  |  |  |
| Product Approved On Remarks |  |  |  |  |
| B-Lite BL-2004 - Type B - Interplex 11-28-2006 |  |  |  |  |
| D-Lite DL-2001 - Type A/C - Interplex 11-28-2006 |  |  |  |  |
| Maverick 051-1003/6-Type A/C - Interplex 11-28-2006 |  |  |  |  |

| Product Category : Barricade Warning Lights |  |  |
| --- | --- | --- |
|  | MATERIAL : 1018M00070 Barricade Warning Light |  |
| APS00014630 Pi-Lit Technology - Mesa, AZ Phone : 949-415-9411 Ext. # : 216 S. Alma School Road #8 Fax : Mesa, AZ 85210 Contact : Jordan Zraick (jordan@pi-lit.com) Additional Info : Mailing Address: 3002 Dow Avenue; Suite 138; Tustin, CA 92780 Product Approved On Remarks BC3.AC.D3.XX D-Cell - Type A/C - Pi-Lit 1-25-2021 BC6.AC.DB.XX.6V Battery-Type A/C- Pi-Lit 1-25-2021 BC6.B.D3.XX D-Cell - Type B - Pi-Lit 1-25-2021 BR3.D.D1.XX.D-Cell - Type D- Pi-Lit 1-25-2021 BS.AC.D1.XX Solar - Type A/C - Pi-Lit 1-25-2021 BS.B.D1.XX/BS.DB.X Solar B - Pi-Lit 1-25-2021 |  |  |
| APS00002150 Protection Services, Inc. - Harrisburg, PA Phone : 949-573-9219 Ext. # : 635 Lucknow Lane Fax : 318-688-9683 Harrisburg, PA 17110 Contact : Denise Sheppard (dsheppard@traffixdevices.com) Additional Info : Contact person's mailing address: 5905 Financial Plaza, Suite 1600; Shreveport, LA 71129 Product Approved On Remarks Protect-O-Flash HI3 - Type B 8- 8-1995 Protect-O-Flash PF-4P-LED - Type A/C 8- 8-1995 |  |  |
| APS00002180 Solar Masters, LLC - Garden Grove, CA Phone : 877-917-6527 Ext. # : 12621 Hoover Street Fax : 714-436-5753 Garden Grove, CA 92841 Contact : Kevin Ryan (kryan@ solar-masters.com) Additional Info : Mailing Address: 3710 industry Ave., Suite #204 Lakewood,CA 90712 Product Approved On Remarks Solar Hazard Flashing - Type A/C 5-14-2003 Solar Hazard Flashing - Type B 5-14-2003 |  |  |
| APS00002190 Star Headlight and Lantern Co. - Avon, NY Phone : 585-226-9787 Ext. # : 455 Rochester Street Fax : Aon, NY 14414 Contact : (CustomerService@star1889.com) Product Approved On Remarks Starlite 747 - Type A/C - StarHedlight 7- 1-2008 |  |  |