Attachment B – Specifications

RFx No.: 3000026112             Title: *REBID* Boat, Motors, & Trailer - LDWF

Line 1: Boat

Brand: Metal Shark or Equal

Model: 29 Defiant Walk-Around or Equal

- 28’-30’ Aluminum Walk-Around Pilot House Style Monohull with Foam Collar

- 18-25 degree Hull Deadrise at Transom

- Maximum 10ft Beam Over-All

- Minimum 2 – Lifting Strakes on Each Side of Hull Between the Chine and Keel

- Between 10,500 lbs - 13,500 lbs on-Trailer Weight

- Less than 13 ft on-Trailer Height

- Trim Tabs with LED Gauges

Fully Enclosed Aluminum Cabin (with a minimum of 3 doors and seating for 3)

- Fatigue-Reducing Cabin Flooring

- Heavy Duty Shock Mitigating Seats (Minimum of 2 inside the cabin)

- Acoustic Noise Reduction within the Cabin

- Windshield Wiper

- Secure Long Gun Storage (minimum of 2 long guns)

- Electronic Panel

- Dimmer Switches for Lights Compass

- HVAC for Cabin

- Minimum 4-12V USB outlets

- 12V to 110V Power Inverter Capable of Running Cabin HVAC and Outlets.

- Minimum 2-GFI 110V Receptacles (Water Proof)

- Breaker Panel

- Stereo with Speakers

Navigation / SONAR / Camera System

- 2 (qty.) - Axiom2 Pro 12 RVM HybridTouch or Equal 12" Multifunction Display with Integrated 1kW Sonar, DV, SV and RealVision Max 3D Sonar and LightHouse North America Chart

- Raymarine RVM-100 RealVision 3D or Equal All-In-One Transom Mount Transducer

- Stainless Steel SS60 Low Profile Through Hull Transducer w/20 Degree Tilted Element

- Raymarine Quantum 2 Q24D Doppler 18" Radar with Power and Data Cable or Equal

- Raymarine AIS 5000 Class A Transponder Bundle or Equal

- AIS4000/AIS5000 VHF Antenna (and fixings)

- Ray73 VHF Radio with GPS, AIS Receiver and Loudhailer Output

- AR200 IP Camera Stabilization Module for Augmented Reality

- FLIR M364C Dual Payload 640 CTV Camera, 24-deg HFoV, 30 Hz with Electronic Zoom and Color Low Light Camera with 30X Electronic Zoom and Joystick Control Unit or Equal

- Newmar Waterproof Hailing Horn

- Raymarine/FLIR All required Cables and Fixings for AIS System or Equal

Fully Walk Around Cabin

- Minimum of 13” Floor (Deck) Space on each side of Cabin

- Minimum of 52” Floor (Deck) Space Extending Aft of Cabin to Rear Transom

- Minimum of 72” Floor (Deck) Space Extending Forward of Cabin Towards Bow

- Anti-Fatigue, Non-Skid Flooring (“Sea-Deck” or Comparable) on Floor (Deck) Space Around Cabin

- Foldable Radar / Camera Mast

- Bow Anchor Storage with Anchor and Chain

- LED Navigation Lights

- Welded Bow Eye

- Minimum of 6 Cleats Total (forward/midship/aft on port and starboard side)

- Grey or Green Foam Impact Resistant Collar around port, starboard sides and bow

- Non-Skid Around Deck of the Interior of the Hull

- Vinyl Lettering only for Exterior of the Hull

- Grab Rail along Cabin

- Bow Rails – Low profile

- Telescopic or Concealable Mounted Swim Ladder

- Stern Tow Post

- Stern Engine Guard

- Bow Push Bumper or Push Capable

- Bilge Pumps

- 3 Fire Extinguishers

- Quad Battery System with Isolator

- Optima Blue Top Batteries

- Shore Power with Charger and Cord

- LED Port/Starboard/Stern Flood Lights

- LED – Remote Searchlight

- LED- Whelen MINI Freedom 21.5” lightbar or Equal

- Whelen Siren or Equal

- Cabin Mounted LED Flush Mounted Emergency Blue Lights (Whelen or equivalent)

- Power Steering

- Mounted EPIRB

- Transhield Wrap

- Manufacturer’s Standard Warranty

Lines 2 and 3: Motor

Brand: Suzuki or Equal

Model: 350HP DF350AMDXX5 or Equal

Motor Features

- 350HP 4 Stroke

- Digital Gauges for Twin Engines

- Pre-Rig and Install Engines

- Hydraulic Steering

- Racor Fuel Water Separators or Equal

- Dual Props

- Manufacturer’s Standard Warranty

Engine Break in procedures completed upon delivery. Motors to be factory installed. All electrical wiring must be terminated, fused, wrapped, and connected.  Hydraulic steering must be installed and tested.  Completed boat must be water tested of all systems and accessories before delivery / pickup. LDWF inspection and water testing required prior to pickup or delivery.

Line 4: Trailer

- Towable Heavy Duty Commercial Trailer 2-5/16” ball (non-fifth wheel or gooseneck)

- Transom Tie Down Straps

- Spare Tire and Rim

- Ladder

- LED Trailer Lights

- Manufacturer’s Standard Warranty

## RIGGING

- All rigging with complete package done by vendor