Scope of Work:

The Contractor shall supply all labor, equipment, and materials to repair the foundation of JESTC Shoot House for the Department of Public Safety (DPS).

Location:

JESTC New Shoot House

1400 West Irene Road

Zachery, LA 70791

Agency Contact:

Lt. Coye Corkern

Coye.Corkern@la.gov

(225) 252-3675

Specifications:

The Contractor shall be responsible for the following:

Exterior Helical Pile Installation – 2.875" Piles (#2)

- Excavate to expose footing and notch flush with wall load.

- Install two 7/8" helical piles to engineered torque specifications.

- Use underpin brackets to transfer structural loads.

- Apply lift as needed to attempt recovery of structural movement (lift/recovery not guaranteed).

- Secure piles and brackets; backfill excavated areas; clean and restore all disturbed areas.

Concrete Cut & Replacement

- Saw cut and remove existing concrete in designated areas.

- Replace concrete to match existing slab thickness and finish.

- Ensure work area is clean and free of debris.

Helical Pile Installation with Slab Brackets – 2.375" Piles (#5)

- Core or chip concrete to install two 3/8" helical piles with fold-out slab brackets.

- Advance piles to meet engineered torque specifications.

- Attempt limited floor leveling via slab lift (full recovery or perfect levelness not guaranteed).

- Backfill holes, patch concrete, and clean the work area.

- DPS shall be responsible for repairing/replacing any disturbed floor coverings.

Polyurethane Foam Injection – Shoot House

- Drill small-diameter holes in foundation slab or designated areas.

- Inject high-density polyurethane foam to fill voids beneath the structure.

- Lift slabs to practical recovery or fill voids only based on soil type and project objectives.

- Replace excavated soil, patch concrete, restore landscaping, and remove debris.

General Terms:

- All corrections for work that is unsatisfactory shall be corrected at the vendor’s expense and must be completed satisfactorily before approval and final payment is made.

- All work shall conform to the requirements of all laws and ordinances in force in the locality in which the work is performed.

- The Contractor shall be fully qualified under any state or local licensing law for contractors in effect at the time and at the location of the work. All work shall be undertaken by experienced workers with proper personal protective equipment using necessary procedures to maintain workplace safety.