**If bidding other than specified, please provide manufacturer’s specs for “or equal” items. **

Scope of Work

The Contractor shall furnish all necessary labor, materials, tools, equipment, and supervision to complete flooring replacement and painting/sheetrock for the following locations at the Louisiana Department of Veterans Affairs (DVA).

The project includes approximately 80–85% of the 8,000 square foot space, consisting of 14 offices, associated storage areas, lobby, inner and outer hallways, conference room, break room, and two bathrooms.

Location:

LA War Veterans Hospital

4739 Hwy 10

Jackson, LA 70748

Agency Contact:

Shannon Sutton

(225) 634-4420

Specifications:

The Contractor shall be responsible for the following:

- Removal of existing glue-down carpet throughout designated areas

- Preparation and resurfacing of all flooring areas to ensure proper installation

- Installation of new carpet flooring in offices and hallways

- Removal and replacement (moving) of furniture in all offices to allow for flooring work

- Installation of new 4-inch cove base along flooring edges

Flooring Material

- Product: Shaw Scoreboard II Carpet or Equal

Specify Brand/Model Bidding: ________________

- Style/Color: 2 Color “First Down 00501”

- Weight: 26 oz. carpet

- Application Areas: Main administrative offices and hallways

Additional Material & Installation Requirements

- Cove Base:

  - 4-inch height

  - Color: To Be Determined (TBD)

- Surface Preparation:

  - All flooring areas will be prepped and resurfaced prior to installation to ensure proper adhesion and finish

- Furniture Handling:

  - All office furniture will be removed and replaced as part of the project

- Demolition:

  - Existing glue-down carpet will be fully removed before installation

Surface Preparation and Repairs

- Remove all existing wallpaper from designated areas

- Patch and repair sheetrock as needed to restore damaged surfaces

- Float walls as required to achieve a smooth and uniform finish

- Sand and prepare all wall surfaces to ensure proper adhesion of paint

- Tape and install protective plastic coverings in all required areas to protect adjacent surfaces

Painting Requirements

- Apply 1 coat of primer to all prepared surfaces

- Apply 2 finish coats of paint as follows:

  - Manufacturer: Sherwin-Williams or Equal

Specify Brand/Model Bidding: ________________

  - Color: Agreeable Grey

  - Finish: Exterior Semi-Gloss

- The conference room may require:

  - A second color, to be determined and approved by the DVA

  - Trim painting, to be completed separately as directed by the DVA

Coordination Requirements

- The Contractor shall perform work in phases, relocating occupants from at least three offices at a time to minimize disruption

- The Contractor shall remove existing cove base as required; the flooring contractor shall reinstall the cove base after completion of painting

General Terms:

- The Contractor shall be responsible for delivery and pickup of materials/supplies needed to complete project.

- Disposal of all materials, and debris shall be the responsibility of the Contractor.