Scope of Work:

The Contractor shall provide all labor, materials, equipment, supervision, and services necessary to remove the existing metal roofing and wall panels and install a new T panel metal roofing and wall system, including associated structural repairs and upgrades for the Eastern LA Mental Health System (ELMHS).

Location:

Eastern Mental Health Hospital

4502 Hwy 951

Jackson, LA 70748

Agency Contact

Jonathan Holley or Dylan Fury

(225) 634-0202

Specifications:

The Contractor shall be responsible for the following:

Demolition

- Remove the existing metal roofing system from:

  - Roof surface

  - Front wall

  - Right-side wall

  - Rear wall

- Properly haul away and dispose of all removed materials in accordance with local regulations.

Structural Repairs

- Inspect roof decking and wall framing upon removal of existing materials.

- Replace rotted, cracked, or otherwise compromised wood slats and framing members as needed to ensure structural integrity.

Roofing System Installation

- Fabricate and install a 26-gauge 2” T panel metal roofing system.

- Panels to be:

  - 26-gauge Galvalume

  - Double lock standing seam

  - Manufactured on-site for precise fitness and performance

- Installation to include:

  - Drip edge

  - Rake trim

  - All required fasteners and sealants

Wall Panel Installation

- Fabricate and install new 2” standing T panel metal wall panels on:

  - Rear wall

  - Right-side wall

- Remove interior wall wood as necessary to open wall bays and accommodate new panel installation.

General Terms/Conditions:

- The Contractor shall maintain a clean and safe work environment throughout the project.

- The Contractor shall perform final cleanup upon completion, removing all debris and construction materials from the site.

- All materials shall be new, of commercial quality, and installed per manufacturer’s specifications.

- Work shall be performed by qualified roofing technicians experienced in metal roof installation.

- Sealants shall be high-performance, weather-resistant types compatible with Galvalume materials.