Scope of Work:

The Contractor shall furnish all labor and materials to repair and renovate the exterior of Home #205 for the Louisiana Department of Health (LDH).

Location:

Pinecrest Developmental Center

136 Tinker Nugent Dr.

Pineville, LA. 71360

Agency Contact

Shannon Aymond

(318) 641-2236

Specifications:

The Contractor shall be responsible for the following:

- Remove the bottom of current plywood siding 24” from the bottom of wall and replace any rotted framework with treated lumber, then replace removed plywood.

- Repair any rotted wood in exposed laminated beams

- Repair or replace any rotted facial with the same material as needed.

- Repair or replace any soffit with the same material as needed.

- Remove all facings then wrap all exposed plywood siding with house wrap.

- Install new LP Trim boards and 12” LP Lap siding. The LP trim boards and Lap siding will be provided by the LDH.

- Caulk all joints and trim with Elastomeric Acrylic Urethane Caulk.

- Paint all exterior with Resilience Exterior Latex Flat or equal. (The stow is not to be included.)

Specify Brand/Model Bidding: __________________

General Terms/Conditions:

- Working hours shall be no earlier than 8:00am to no later than 5:00pm Mon-Fri.

- The Contractor shall supply all necessary materials and labor described in the Specifications section above.

- Disposal of all materials and debris shall be the responsibility of the Contractor.

- The Contractor shall coordinate all work with agency. This includes delivery, storage, and work.

- The Contractor shall not leave tools unattended at any time as clients might injure themselves or others.