**If bidding other than specified, please provide manufacturer’s specs for “or equal” items. **

Scope of Work:

The Contractor shall provide all labor, materials, equipment, supervision, and services necessary to complete a full roof replacement, including removal of the existing roofing system and installation of a new roofing system, along with replacement of soffit and facia around the entire perimeter of the building for the Eastern LA Mental Health System (ELMHS).

Location:

Eastern Mental Health Forensic Building

4502 Hwy 951

Jackson, LA 70748

Agency Contact

Jonathan Holley or Dylan Fury

(225) 634-0202

Specifications:

The Contractor shall be responsible for the following:

Demolition

- Remove the existing shingle roofing system down to the roof deck.

- Remove all existing soffit and fascia around the building perimeter.

- Haul off and properly dispose of all removed materials and debris in accordance with local regulations.

Installation

- Fabricate and install a 24-gauge Galvalume T-panel standing seam roofing system.

- Panels to be roll-formed on-site to ensure proper fit and minimize seams.

- Install per manufacturer specifications and industry standards.

The following items are included in the base scope:

- 24-gauge Galvalume standing seam panels

- Synthetic felt underlayment

- Drip edge

- Starter strip

- Hip and ridge cap system

- Valley metal

- Pipe jacks and flashing components

- Roof ventilation (750 vents or Equal system)

Specify Ventilation System Bidding: ____________________

- New soffit and fascia installed around the full perimeter of the building

- All required fasteners, sealants, and accessories

General Terms/Conditions:

- The Contractor shall coordinate all work with Owner and facility operations.

- All equipment shall be new and installed according to the manufacturer’s recommendations.

- Work shall comply with all applicable Louisiana codes and FP&C standards.

- The Contractor shall maintain site safety, security, and cleanliness.