# Attachment B – Specifications

RFx #3000026250

# 2027 F-650 Gas, Regular Cab Base (F6A), or equal

## GENERAL DESCRIPTION

These specifications describe the minimum requirements for one new box truck. This vehicle shall be furnished complete with all necessary operating accessories, equipment, and manuals. The vehicle shall be new, unused, and of the manufacturer’s latest production model meeting or exceeding the requirements listed herein.

A 2027 F-650 Gas, Regular Cab Base (F6A) was used to develop the specifications below and establish equivalency criteria. Equipment of similar style, type, character, quality, features, and purpose conforming to the following detailed requirements/specifications will be considered. For evaluation purposes, bidder's proposing an exception/equivalent option/feature to those specified herein, may be required to provide manufacturer/product information (catalogue sheets, detailed specifications, pictures, etc.). This information will be evaluated against the minimum requirements of this specification. Proposed submittals that are determined not to be equivalent to the established criteria will be rejected.

## Equipment and Specifications

## Dimensions:

GCWR: 37,000 lbs. • Front brake diameter: 15.0" • Rear brake diameter: 15.0" • Driver distance from axle: 54.4" * Vehicle body length: 308.0" • Vehicle body width: 96.7" • Vehicle body height: 94.7" * Wheelbase: 194.0" * Turning radius (to curb): 26.6' • Front track: 80.5" • Rear track: 72.6" * Cab to axle: 120.0" • Rear tire outside width: 96.0" * Axle to end of frame: 75.0" • Frame section modulus: 12.6 cu.in. • Frame yield strength (psi): 50000.0 • Frame rail depth: 10.1" • Frame rail width: 3.1" • Frame rail thickness: .3" • Frame rail section: 9.5" • Front bumper to front axle: 39.0" • Headroom first-row: 40.7" • Leg room first-row: 41.4" • Shoulder room first-row: 68.0" • Hip room first-row: 67.6"

Powertrain:

Spark ignition system • 7.3L V-8 variable valve control, engine with 335HP • Engine cylinders: V-8 • Injection Type: sequential MPI • Horsepower: 335 HP@3750 RPM • Torque: 468 lb.-ft.@3750 RPM • Radiator • PRNDM shift indicator • 75 mph speed limiter • 6-speed automatic • Rear-wheel drive • Recommended fuel: regular unleaded • Low-speed ABS traction control • Standard rear differential

Fuel Economy and Emissions

Federal emissions

Suspension and Handling

Standard ride suspension • Gas-pressurized front shock absorbers

Drivability

Hydraulic disc brakes • Front and rear ventilated disc brakes • 4-wheel antilock (ABS) brakes • Four channel ABS brakes • Hill start assist • Rigid axle front suspension • Leaf spring front suspension • Tapered leaf front suspension • Rigid axle rear suspension • Leaf spring rear suspension • Hydraulic power-assist steering system • Re-circulating ball steering • 2- wheel steering system

### Body Exterior

Side assist steps • 2 doors • Monotone paint • Black side window trim • Black door handles • Black windshield trim • Black front bumper • 2 front tow hooks • Straight front bumper ends • Black grille • Hood mounted grille • Convex spotter in driver and passenger side door mirrors • Trailer style side mirrors • Black door mirrors • English measure truck 279/82R22.5 AS BSW front and rear tires • 22.5 x 8.25-inch front and dual rear wheels • White front wheels • Steel front wheels • White rear wheels • Steel rear wheels • Disc rear wheel

### Convenience

Manual door locks • Cruise control with steering wheel mounted controls • Manual first-row windows • Partial floor console • Driver foot rest • Fixed rear windshield • Front beverage holders • Standard glove box • 2 beverage holders • Instrument panel bin • Dashboard storage • Trip computer • Dual electric horn • Steering column lever gearshift location

### Comfort

Manual climate control • Cloth headliner material • Full headliner coverage • Full vinyl floor covering • Full floor coverage

Urethane steering wheel • Manual tilting steering wheel • Manual telescopic steering wheel

Seats and Trim

Seating capacity: 2 • Driver seat armrest • Front passenger seat armrest • Bucket driver seat • Folding driver seat back • Low driver seat back • 4 driver seats way-direction • Manual fore/aft • Bucket passenger seat • Fixed passenger seat back • Low passenger seat back • 2 passenger seats way-direction • Manual fore/aft • Vinyl front seat upholstery

### Entertainment Features

AM/FM • In-vehicle audio • AM radio • FM radio • Seek scan • Auxiliary input jack • External memory control • Standard grade speakers • Speakers number: 2  • Steering wheel mounted audio controls • Wireless audio streaming • Fixed audio antenna

Lighting, Visibility and Instrumentation

Analog instrumentation display • Trip odometer • Digital clock • Driver information center • Tachometer • Oil pressure gauge • Engine/electric motor temperature gauge • Oil temperature gauge • Transmission fluid temperature gauge • Engine hour meter • Light tinted windows • Reflector headlights • Halogen headlights • Auto on/off headlight control • Multiple enclosed headlights • Delay-off headlights • DRL preference setting • Variable intermittent front windshield wipers • Speed sensitive wipers • Front reading lights • Variable instrument panel light • Daytime running lights • Cab clearance lights • Delay interior courtesy lights

### Technology and Telematics

Handsfree wireless device connectivity • 1 USB port

Safety and Security

Vehicle tracker • Electronic stability control system

Dimensions

- General Weights:

  - Curb Weight: 8,940 lbs

  - Front GAWR: 8,500 lbs

  - GVWR: 26,000 lbs

    - Rear Curb Weight: 3,426 lbs

    - Rear GAWR: 17,500 lbs

    - GCWR: 37,000 lbs

    - Trailer Weights:

      - GCWR: 37,000 lbs

      - Weights:

            - Max RBM: 665,500 lbs

            - Normal RBM: 632,000 lbs

            - Front Weights:

            - Front Curb Weight: 5,514 lbs

            - Spring Rating Front: 8,500 lbs

            - Axle Capacity Front: 8,500 lbs

            - Tire/Wheel Capacity Front: 13,220 lbs

            - Rear Weights:

            - Axle Capacity Rear: 17,500 lbs

            - Tire/Wheel Capacity Rear: 24,040 lbs

            - Spring Rating Rear: 19,000 lbs

            - Exterior Measurements:

            - Vehicle Body Length: 308”

            - Vehicle Body Width: 96.7”

            - Front Brake Diameter: 15”

            - Vehicle Body Height: 94.74”

            - Rear Brake Diameter: 15”

            - Rear Frame Height Loaded: 36.5”

            - Turning Radius (to bumper): 28.2”

            - Width – Mirrors Extended: 121.9”

            - Rear Track: 72.6”

            - Turning Radius (to curb): 26.6’

            - Rear Tire Outside width: 96”

            - Frame Section Modulus: 12.6 cubic inches

            - Frame Rail Depth: 10.1”

            - Frame Rail Thickness: 0.3”

            - Front Bumper to Front Axle: 39”

            - Front Frame Height Loaded: 35.6”

            - Front Frame Height Unloaded: 36.8”

            - Rear Frame Height Unloaded: 37.8”

            - Width Mirrors Folded: 90.7”

            - Front Track: 80.5”

            - Driver Distance from Axle: 54.4”

            - Cab to Axle: 120”

            - Axle to end of Frame: 75.0”

            - Frame Yield Strength (psi): 50,000

            - Frame Rail Width: 3.1”

            - Frame Rail Section: 9.5”

            - Headroom:

            - Headroom First-row: 40.7”

            - Legroom:

            - Leg Room First-row: 41.4”

            - Shoulder Room:

            - Shoulder Room First-row: 68.0”

            - Hip Room:

            - Hip Room First-row: 67.6”

Powertrain

- Engine:

- Engine: 7.3L V8 Variable valve control, engine with 335HP

- Engine Cylinders: V8

- Engine Locations: Fronted mounted engine

- Engine Mounting Direction: Longitudinal mounted engine

- Cylinder Head Material: Aluminum cylinder head

- Valves per cylinder: 2

- Injection Type: Sequential MPI

- Ignition: Spark ignition system

- Engine Block Material: Iron engine block

- Engine Specifications:

- Displacement: 7.3L

- Cubic Inch Bore: 4.22”

- Compression Ratio: 10.5

- Cubic Centimeters: 445

- Stroke: 3.98”

- Engine Power:

- Horse Power: 335 HP at 3,750 RPM

- Torque: 468 lb-ft at 3,750 RPM

- Alternator

- Alternator Amps: 210A

- Alternator Type: Heavy-Duty

- Battery:

- Battery Type: Lead acid battery

- Battery Step: Battery Step

- Battery Rating: 900CCA

- Battery Location: Battery location forward right

- Battery Run Down Protection: Battery Run Down Protection

- Engine Extras:

- Radiator: Radiator

- Shift Indicator: PRNDM shift indicator

- Transmission:

- Transmission: 6-speed automatic

- Speed limiter: 75 mph speed limiter

- Lock-up transmission: Lock-up transmission

- First gear ratio: 3.974

- Second gear ratio: 2.318

- Third gear ratio: 1.516

- Fourth gear ratio: 1.149

- Fifth gear ratio: 0.858

- Sixth gear ratio: 0.674

- Reverse gear ratio: 3.128

- Stall ratio: 2

- Transmission oil cooler: Transmission oil cooler

- Transmission electronic control: Transmission electronic control

- Overdrive transmission: Overdrive transmission

- Selectable mode transmission: Selectable mode transmission

- Drive Type

- Drive type: Rear-wheel drive

- Drivetrain

- Axle ratio: 6.14

- Axle speed: Single axle speed

- Exhaust:

- Tailpipe: Non stainless steel single exhaust

- Additional muffler location: right

- Additional tailpipe location: right

- Additional muffler type: horizontal

- Additional tailpipe type: horizontal

- Fuel:

- Fuel type: Regular unleaded

- Fuel Tank:

- Front left fuel tank shape: rectangular

- Front left fuel tank capacity: 50.01 US gallons

- Drive Feature:

- Traction control: Low-speed ABS traction control

- Differential: Standard rear differential

### Fuel Economy and Emissions

- Emissions

- Emissions: Federal emissions

### Suspension and Handling

- Suspension

- Suspension: Standard ride suspension

- Front shock absorbers: Gas-pressurized front shock absorbers

### Driveability

- Brakes:

- Brake type: Hydraulic disc brakes

- ABS brakes: Four channel ABS brakes

- Ventilated brakes: Front and rear ventilated disc brakes

- ABS brakes: 4-wheel antilock (ABS) brakes

- Brake Assistance

- Hill start assist: Hill start assist

- Front Suspension:

- Suspension ride type front: Rigid axle front suspension

- Suspension type front: Leaf spring front suspension

- Front Spring:

- Regular front springs: Regular front springs

- Springs front: Tapered leaf front suspension

- Rear Spring:

- Springs rear: Multi-leaf rear suspension

- Auxiliary rear spring: Rubber auxiliary rear spring

- Rear springs: Regular grade rear springs

- Rear Suspension:

- Suspension type rear: Leaf spring rear suspension

- Suspension ride type rear: Rigid axle rear suspension

- Steering type number of wheels: 2-wheel steering system

Exterior

- Front Wheels:

- Front wheels diameter: 22.5”

- Front wheels width: 8.3”

- Rear Wheels:

- Rear wheels diameter: 22.5"

- Rear wheels width: 8.3"

- Front Tires:

- Aspect: 82

- Sidewalls: BSW

- Type: English measure truck tire

- Front Wheel – RPM: 497

- Diameter: 22.5”

- Tread: AS

- Width: 279mm

- Rear Tires:

- Aspect: 82

- Sidewalls: BSW

- Type: English measure truck tire

- Front Wheel – RPM: 497

- Diameter: 22.5”

- Tread: AS

- Width: 279mm

Body Exterior

- Exterior Features:

- Number of doors: 2 doors

- Front splash guards: Front splash guards

- Side steps: Side assist steps

- Frame:

- Clean CA frame: Clean CA frame

- Body:

- Body panels: Composite and galvanized steel body panels

- Mirrors:

- Convex spotter: Convex spotter in driver and passenger side door mirrors

- Tires:

- Front tires LT load rating: H

- Rear tires LT load rating: H

- Wheels:

- Front wheel type: Disc front wheel

- Number of front wheel studs: 10 front wheel studs

- Dual rear wheels: Dual rear wheels

- Number of rear wheel studs: 10 rear wheel studs

### Convenience

- Door Locks:

- Door locks: Manual door locks

- Cruise Control:

- Cruise control: Cruise control with steering wheel mounted controls

- Exterior Mirrors:

- Folding door mirrors: Manual folding door mirrors

- Heated door mirrors: Heated driver and passenger side door mirrors

- Front Side Windows:

- First-row windows: Manual first-row windows

- Floor Console:

- Floor console: Partial floor console

- Passenger Visor:

- Visor passenger mirror: Passenger visor mirror

- Power Outlets:

- 12V power outlets: Two 12V power outlets

- Pedals:

- Driver foot rest: Driver foot rest

- Rear Windshield:

- Rear windshield: Fixed rear windshield

- Storage:

- Number of beverage holders: 2 beverage holders

- Glove box: Standard glove box

- Dashboard storage: Dashboard storage

- Beverage holders: Front beverage holders

- Instrument panel storage: Instrument panel bin

- Miscellaneous:

- Trip computer: Trip computer

- Gearshift location: Steering column lever gearshift location

- Horn: Dual electric horn

Comfort

- Climate Control

- Climate control: Manual climate control

- Headliner:

- Headliner material: Cloth headliner material

- Headliner coverage: Full headliner coverage

- Floor Trim:

- Floor covering: Full vinyl floor covering

- Floor coverage: Full floor coverage

- Steering Wheel:

- Steering wheel material: Urethane steering wheel

- Steering wheel telescopic: Manual telescopic steering wheel

- Steering wheel tilt: Manual tilting steering wheel

### Seats and Trim

- Seat Capacity:

- Seating capacity: 2

- Front Seats:

- Front seat type: Bucket driver seat

- Driver seat back: Folding driver seat back

- Driver seats way-direction: 4 driver seats way-direction

- Passenger seat back: Fixed passenger seat back

- Passenger seats way-direction: 2 passenger seats way-direction

- Armrests front driver: Driver seat armrest

- Front seat type: Bucket passenger seat

- Driver seat back type: Low driver seat back

- Driver seat fore/aft: Manual fore/aft

- Passenger seat back type: Low passenger seat back

- Passenger seat fore/aft: Manual fore/aft

- Armrests front passenger: Front passenger seat armrest

- Front seatback upholstery: Carpet front seatback upholstery

Entertainment Features

- Radio Features:

- Aux input jack: Auxiliary input jack

- External memory: External memory control

- Seek scan: Seek scan

- Speakers:

- Speakers: Standard grade speakers

- Speakers number: 2

- Audio Features:

- Steering mounted audio control: Steering wheel mounted audio controls

- Wireless streaming: Wireless audio streaming

### Lighting, Visibility and Instrumentation

- Instrumentation:

- Trip odometer: Trip odometer

- Instrumentation display: Analog instrumentation display

- Instrumentation Displays:

- Driver information center: Driver information center

- Clock: Digital clock

- Instrumentation Gauges:

- Tachometer: Tachometer

- Oil temperature gauge: Oil temperature gauge

- Engine/electric motor temperature gauge: Engine/electric motor temperature gauge

- Oil pressure gauge: Oil pressure gauge

- Transmission temperature gauge: Transmission fluid temperature gauge

- Engine hour meter: Engine hour meter

- Instrumentation Warnings:

- Oil pressure warning: Oil pressure warning

- Low brake fluid warning: Low brake fluid warning

- Headlights on reminder: Headlights on reminder

- Door ajar warning: Door ajar warning

- Transmission fluid temperature warning: Transmission fluid temperature warning

- Low fuel warning: Low fuel warning

- Battery charge warning: Battery charge warning

- Key in vehicle warning: Key in vehicle warning

- Service interval warning: Service interval warning

- Glass:

- Tinted windows: Light tinted windows

- Headlights:

- Headlights: Halogen headlights

- Headlight type: Reflector headlights

- Auto headlights: Auto on/off headlight control

- Multiple headlights: Multiple enclosed headlights

- Delay off headlights: Delay off headlights

- DRL preference setting: DRL preference setting

- Front Windshield:

- Wipers: Variable intermittent front windshield wipers

- Speed sensitive wipers: Speed sensitive wipers

- Interior Lighting:

- Variable panel light: Variable instrument panel light

- Front reading lights: Front reading lights

- Lights:

- Running lights: Daytime running lights

- Clearance lights: Cab clearance lights

- Interior courtesy lights: Delay interior courtesy lights

Technology and Telematics

- Connectivity:

- Handsfree: Handsfree wireless device connectivity

- USB Ports:

- USB ports: 1 USB port

Safety and Security

- Security System:

- Vehicle tracker: Vehicle tracker

- Traction Control:

- Electronic stability control: Electronic stability control system

# Van Body Specifications

- One Lyncoach 16’L x 96”H x 96”W Aluminum Dry Freight Van Body, or equal

- Dimensions:

  - Length: 15'10" interior

  - Height: 96 1/4" interior

  - Width: 90 1/4" interior

- Door opening: 88"W x 88-1/4"H

  - Exterior: 16'5"

  - Exterior: 103 3/8"

  - Exterior: 96"

- Front: Radius design with stainless steel corner posts.

- Exterior: Pre-painted white .040 aluminum.

- Roof: .032 aluminum, one piece, pre-tensioned with roof bows on 24" centers.

- Rear Door: Composite, twin cable, overhead roll-up door. Door opening: 88"w  x 88-1/4"H

- Interior Lining: 3/8” plywood lining full height on the side walls and ½” Plywood on the front wall.

- Floor: 1 1/8" laminated hardwood floor

- Understructure: 4" steel I-beam longsills, 3" crossmembers on 12" centers.

- Lights: Lights and reflectors per FMVSS #108 specifications.  Interior dome light with switch.

- Mud Flaps: One pair of anti-sail mud flaps.

- Installation: Fully installed on your truck chassis with 5/8" diameter U-Bolts and 20 yr. celcure wood frame cushions.

# Lift Gate Specifications

- One Maxon Tuckunder Liftgate Model TE-25 Liftgate, Manual Control, Gravity Down, or equal.

- Capacity: 2,500 lbs.

- Platform Size: 80”W x 48”D, Wedge style Platform, Platform is constructed of Steel

- Steel Dock Bumpers with steps

- Paint Black

BIDDER’S EXCEPTIONS

Instructions: Bidder should note all exceptions in space provided below. List the specification in the column to the left and the exception in the column to the right. Responses may be typed or hand-written. Handwritten responses must be legible. If additional space is needed, please print a duplicate copy of this sheet. "Bidder’s Exceptions" page(s) should be returned with the bid submittal.

Specification/Detail                                                                                 Exception

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________