RFx: 3000026229					Title:*MandJobsiteVst* Grounds Maintenance-DMA

Attachment B – Specification

Jackson Barracks Lawn Service

## Scope:

To provide the services of a qualified Contractor with all labor, transportation, equipment and insurance to furnish all operations connected with this grass cutting contract. The work shall include the furnishing of all necessary labor, supervision, tools, equipment, materials, supplies, miscellaneous items, fuel, equipment maintenance, storage/shop facilities, licensing fees, permits and other resources required and incidental to the cutting of the grass as outlined in these specifications. The services required include trimming, mowing, edging, bagging and the application of herbicides and fertilizers.

- All fine cut areas will first be cleaned of debris and loose objects that could become projectiles when struck by mowers. Grass cutting height will be per industry standards in accordance with the predominant grass species in the area to be cut but is not to exceed 2 ½ inches at the height of the mowing season for St Augustine grass. No more than 1/3 of the grass blade is to be removed at one time. Fine cut areas that become "scalped" due to Contractor actions shall be reseeded and maintained at no additional cost to the State. Discharged grass clippings will not be allowed to form windows or be left in clumps. Each moving cycle includes mowing, trimming, edging and debris removal.

- Fence lines, asphalt, sidewalks, driveways and other such concrete pads shall be kept weed, grass and moss free by hand or use of herbicides. Trimming and edging around lawn perimeters, sidewalks, driveways parking lots buildings and bedded areas shall be done with each mowing with proper removal of associated debris. All fence lines, sidewalks, curbs, and other concrete edges will be edged and trimmed with a mechanical type of edger in conjunction with each mowing. The edging groove is not to exceed one inch. Vacuum or blow clean all sidewalks, streets, driveways and/or concrete areas affected by work. All debris shall be removed from the site. These operations are to be done consecutively and will be completed before moving to subsequent areas. The State will not pay for mowing unless all operations have been satisfactorily completed as specified. All grass clippings are to be removed from the sidewalks, driveway and streets, and are not to be left in place overnight. Apply a selective herbicide to gravel/shell parking areas and fence lines to control the growth of grass/weeds. Clean up will consist of removing wastepaper, limbs, branches, litter and debris. All such waste may be deposited in State owned dumpsters located on site.

- Pick up and remove all leaves and debris in parking lots, streets and sidewalks starting in autumn (when leaves on surrounding trees start to fall) and spring (when oak trees lose their leaves). Perform this service in coordination with the Agency.

- Control weeds in all landscape turf areas with pre-emergent herbicide three times a year (March, August, and October) and contact herbicide as needed in Areas A-1, A-2, B, C around the Museum area and conference center, and area E Housing area.

Frequency: All areas of the installation are to be visited 52 times per year. Visits may consist of complete cuts, spot cuts, leaf control, and debris removal to be coordinated with the local facility coordinator. Each visit should be completed within a 4 day timeframe.

Normal Cutting Days: Grass will be cut as specified herein. Work should be scheduled Weekdays from 8:00 am to 6:00 pm if needed weekend work to be authorized by installation manager. No cutting will occur within 24-hours of the last rainfall of 1 inch in 1 day without checking with the local facility coordinator. Any other variation in the schedule due to weather conditions shall be requested from the contracting officer. Operational requirements of the facility may cause the Contractor to adjust the established cutting schedules with limited advance notification.

Equipment: The Contractor will provide a listing of equipment to be used on site and coordinate with the facility manager to ensure proper type equipment is utilized. This listing should be provided prior to awarding of the contract. Rotary mowers may be used on fine cut areas and equipped with discharge shields to control flying debris discharged during the cut. All equipment that is used in the fine cut areas must be equipped with turf type tires. Trenching and tractor drawn curb dressers are not acceptable means of edging and trimming. Flail moving equipment will not be used in fine cut areas.

Coordination: The Contractor and the local facility coordinator shall together inspect the grounds after each cut to verify contract requirements.

Description of Areas See Attachment C - Site Map - Page 1:

Area A-1

Area A-1 is a total of approximately 15 acres. Approximately 7 acres on the west side of the area should be considered for grass maintenance for this contract. The drainage ditch that runs from south to north is the Iine. Area A-1 is to include all the cutting, edging, trimming and cleaning of grass up to and around all structures aside from construction areas at the time. Area A-1 will also include the levee from Angela St to Delery St as well as the other perimeter from the brick fence to the Delery St from the levee to St Claude Ave. Additionally; Area A-1 includes the levee which runs along the Mississippi River from the east wall to the west wall.

Area A-2

Area A-2 is a total of approximately 17 acres. The area on the west side of Grant Street should be considered for grass maintenance for this contract. Area A-2 is to include cutting, edging, trimming and cleaning of all grass up to and around all structures aside from construction areas at the time. Area A-2 Is to also include the outer perimeter along Delery Street to the west gate adjacent to Building 39 near the traffic circle.

Area B

Area B is a total of approximately 17 acres. Area B is to include cutting, edging, trimming and cleaning of all grass up to and around all structures aside from construction areas at the time. Area B is to also include the neutral ground on St Claude Ave from Delery St to Angela St as well as the outer perimeter from the brick fence to St Claude Ave. Area B is to also include the outer perimeter along Delery Street to St Claude Ave.

Area C

Area C is a total of approximately 22 acres. Area C is to include cutting edging, trimming and cleaning of all grass up to and around all structures aside from construction areas at the time. Area C will also include the outer perimeter from the brick fence to Claiborne Ave from Delery St to Angela St as well as the neutral ground from Delery St to Angela St. Area C Is to also include the outer perimeter along Angela Street to Claiborne Ave.

Area D

Area D is a total of approximately 30 acres. Area D includes cutting, edging, trimming and cleaning of all grass up to and around all structures aside from construction areas at the time. Area D will also include the outer perimeter from the brick fence to Claiborne Ave from Delery St to Angela St. Area D is to also include the outer perimeter along Angela Street to Post Engineer Complex.

Area E

Area E approximately 15 acres. Area E includes cutting, edging, trimming and cleaning of all grass up to and around all structures aside from construction areas at the time. Area E will also include the outer perimeter from the fence line to Florida Ave from the fence to Dubreuil St and along the back fence line on Florida Ave also the retention pond in Area E. Area E is to also include the outer perimeter along Angela Street to Florida Ave on the east and Dubreuil Street from Orin Street to Florida Ave in the west.

Contractor Responsibilities:

The Contractor will provide his own labor, tools, equipment, transportation, etc. The Contractor will dispose of all trimmings and dead growth. Environmentally approved vegetation killers are permissible in all areas to the extent that they do not adversely affect the plants and trees which have been planted and must be approved by Louisiana Military Department prior to application.

Identification:

Contractor personnel will carry Contractor identification at all times while on Louisiana Military Department property. Contractor personnel will show identification when asked by Louisiana Military Department employees and security personnel. Photo identification, proof of insurance and inspection sticker is required for access to facility.

Indemnification:

The Contractor agrees to hold harmless and indemnify LOUISIANA MILITARY DEPARTMENT against any and all claims, suits, damages, costs, or legal expenses as a result of bodily injury or property damage resulting from the negligence of the Contractor. All policies of insurance carried by the Contractor shall be written as primary policies, not contributing with and not in excess of insurance coverage which LOUISIANA MILITARY DEPARTMENT may carry.

Non-Performance

  - In the event the Contractor fails to accomplish any task under this scope of work Louisiana Military Department will provide reasonable notice to take corrective action. If the Contractor does not perform the service, Louisiana Military Department may, at its option, cause the non-performed tasks to be accomplished through another source and deduct the cost of such from the amount normally due to the Contractor for that monthly period.

  - In the event the Contractor fails to accomplish certain tasks, and the delay in taking corrective action results in a missed or partial service, Louisiana Military Department may result in nonpayment.

JACKSON BARRACKS LAWN SERVICE SPECIFICATIONS

52 Visits Per Year

February through November 44 Visits

1.	Mow the grass

- Pick-Up debris on grounds as necessary

- Weed-eat around buildings, trees, and all other obstacles

- Edge sidewalks and curbs

- Blow off cut grass from sidewalks, parking lots and drives.

- Leaf Removal as Necessary.

Attachment B - Specifications

December and January 8 Visits

        - Mow the grass

        - Pick-Up debris on grounds as necessary

        - Weed-eat around buildings, trees, and all other obstacles

        - Edge sidewalks and curbs

        - Blow off cut grass from sidewalks, parking lots and drives.

        - Leaf removal as necessary.

Once a Month

- De-weed sidewalks, parking lots, and drives with herbicide for weed control

- Apply herbicide to fence for weed control