Attachment B - Specifications

Mattresses and Box Springs - Statewide

RFx #3000026215 		T # 90943

Open flame mattress flammability standard:

The U.S. Federal Government has standards regarding the flammability of mattresses when exposed to an open flame. The U.S. Consumer Product Safety Commission has finalized a safety standard for mattresses/foundation sets. The standard is codified as 16 CFR part 1633.

All mattresses/foundations bid under this Invitation to Bid must meet safety standard 16 CFR part 1633.

A label must be permanently affixed to the mattresses confirming compliance with this law. The label must contain the following:

Manufacturer’s Company Name

Street Address

City and State

Month and Year of Manufacturer Prototype

The label must contain the following statement:

“This mattress meets the requirements of 16 CFR part 1633 – Standard for the Flammability (Open Flame) of Mattress Sets -- for its intended use.”

Packaging and Labeling

Mattress Packaging:

Mattresses shall be clean and free of all defects. Mattresses shall be completely covered and sealed with a minimum of 4 mil plastic material.

Mattress Labeling:

Labels shall be in accordance with the State and Federal laws and shall include the percentages of each material(s) used. The manufacturer’s name and date of manufacture shall be included on the label.

# Specifications for Lines 1 through 4:

Hospital/Dormitory Mattress, Polyfoam

Foam Core: 7" 1.5 lbs. density per cubic foot in a 34-36 lb. compression as established by Indentation Load (Force) Deflection (ILD) tests. Mattress must meet 16 CFR part 1633.

Ventilators: not less than 3/4” diameter perforated brass are to be installed in all institutional mattresses.

Mattress Ticking: as specified on each line – shall be 210 Denier or polyester navy fluid proof and anti-microbial sewn inverted with seam along center of border.

# Specifications for Lines 5 through 8:

Residential/Dormitory Mattress

Innerspring, 6" 312 bonnell type coils, 13 gauge wire with a minimum of 6 gauge border rod coils.

Double Sided Construction: The below are to be placed on top & bottom of spring unit above.

Padding: 1.5 ounce resin bonded polyester. 1.5” fire foam pad, 1.5 lbs density, 34-36 lb. ILD.

Cover: Navy FR laminated nylon or polyester fluid proof cover to meet 16 CFR part 1633

Border: Matching laminated fire retardant border to encapsulate entire outer edge on mattress sewn with FR (Kevlar) thread.

Flange: flange material to be at least 6" typar non woven or similar construction sewn to quilted panels , whereby the panels can be stapled to each perimeter coil of spring unit.

Tape: Mod acrylic or equal, 1 1/4 “ minimum width

# Specifications for Lines 9 through 11:

Box Foundations:

Foundation perimeter border ticking. Daiwabo rayon FR corona viscose ray and silicon dioxide material with flash point parameters or equal; colored and degraded at 260 degrees C/ignited at 400-470 degree C.

Foundation Top Panel Ticking:  FR rayon/polyester/acrylic based nonwoven fire retardant filler cloth or equal.

Dormitory Foundation: 9" all wood heavy duty solid foundation with minimum of 5 cross braces and center support rails.

Foundation Insulator: resin bonded polyester insulator barrier or ect 32 flute card board stapled around the entire perimeter of the foundation unit.

Dust Cover: Medium density nonwoven cover.

Corner Guards: Each corner will be protected with a heavy duty plastic corner guard of high impact plastic material secured by heavy duty staples.

Specifications for Lines 12 through 14:

Hotel plush double-sided mattress set

Mattress:

6"   312 Coil 13.0 gauge with perimeter edge support

2 oz. with insulator pad on both sides.

1.8 lbs. 34 ILD foam, 1 ½” thick on top of each fiber layer.

Quilted Panel: 1” 1.5lbs 15 ild  rolled foam, 1.0oz Struto fiber rolled, .5 oz. quilted backing, medium  FR high-loft  fire barrier , stretch knit cover.

Border:  Matching cover FR  laminated /quilted with fire barrier

Flammability standard: 16 CFR part 1633

Finish size: 12” thick

Foundation: Frame to be poplar or equal

Cardboard Topper: ECT 32 c-fluted corrugated cardboard or 2oz resin bonded polyester pad

Top panel non-woven solid FR polyester fabric.

Quilt Backing:  94” ½ oz. white spun bond

Dust Cover: 1.25 oz. black spun bond

Finish Size:  9” thick

Specifications for Lines 14 through 16:

Hotel plush double-sided mattress set

Mattress: 6” 312 Coil 13.0 gauge with perimeter edge support

2 oz. with insulator pad on both sides.

1.8 lbs. 34 ILD foam, 1 ½” thick on top of each fiber layer.

Quilted Panel:

1” 1.5lbs 15 ild rolled foam, 1.0oz Struto fiber rolled, .5oz. quilted backing, medium FR high-loft fire barrier, stretch knit cover.

Border: Matching cover FR laminated / quilted with fire barrier

Flammability standard: 16 CFR part 1633

Finish Size: 12” thick

Foundation:

Frame to be poplar or equal

Cardboard Topper: ECT 32 c-fluted corrugated cardboard or 2oz resin bonded polyester pad

Top panel non-woven solid FR polyester fabric.

Quilt Backing: 94” ½ oz. white spun bond

Dust Cover: 1.25 ox. Black spun bond

Finish Size: 9” thick