Scope of Work:

The Contractor shall supply all labor, equipment, and materials to repair the foundation of JESTC Shoot House for the Department of Public Safety (DPS).

Location:

JESTC New Shoot House

1400 West Irene Road

Zachery, LA 70791

Agency Contact:

Lt. Coye Corkern

Coye.Corkern@la.gov

(225) 252-3675

Specifications:

The Contractor shall be responsible for the following:

Helical Pile Installation and Foundation Underpinning

- Install 20 exterior 2.875-inch helical piles with underpinning brackets to stabilize the affected foundation areas.

  - Excavation required to expose the existing footing.

  - Notching the footing flush with the wall load as required.

  - Installation of 2 7/8-inch helical piles to the required torque and depth in accordance with engineering specifications.

  - Installation of underpinning brackets to transfer structural load to the piles.

  - Application of lift, as necessary, in an attempt to recover settlement conditions. Lift or recovery is not guaranteed.

  - Securing all installed piles and brackets upon completion.

  - Backfilling excavated areas and restoration/cleanup of disturbed work areas.

Concrete Cut, Removal, and Replacement

- Perform concrete cutting, removal, and replacement necessary to facilitate foundation repair operations.

  - Saw cutting and removal of existing concrete as required.

  - Replacement and finishing of disturbed concrete areas to match existing conditions as closely as practical.

Polyurethane Foam Injection and Void Filling

- Provide polyurethane foam injection services beneath the structure slab and designated areas to fill subsurface voids and assist in limiting future movement.

  - Drilling small-diameter access holes in the slab or designated work areas.

  - Injection of polyurethane foam beneath the slab to fill voids.

  - Attempted slab lift and stabilization where practical.

  - Replacement of soil in excavated areas.

  - Restoration of concrete and disturbed areas.

  - Removal and disposal of all work debris.

Mobilization, Engineering, and Permitting

- Provide all mobilization and administrative services required to initiate and perform the project.

  - Site-specific engineering related to the repair plan.

  - Mobilization of personnel and specialized equipment.

  - Applicable permitting fees.

  - Project coordination and administrative setup costs.

General Terms:

- All corrections for work that is unsatisfactory shall be corrected at the vendor’s expense and must be completed satisfactorily before approval and final payment is made.

- All work shall conform to the requirements of all laws and ordinances in force in the locality in which the work is performed.

- The Contractor shall be fully qualified under any state or local licensing law for contractors in effect at the time and at the location of the work. All work shall be undertaken by experienced workers with proper personal protective equipment using necessary procedures to maintain workplace safety.