**If bidding other than specified, please provide manufacturer’s specs for “or equal” items. **

Scope of Work:

The Contractor shall furnish all labor, materials, equipment, tools, supervision, and services necessary to remove and replace approximately 6,000 sq. ft. of roofing for the Department of Health (LDH).

Location:

Pinecrest Supports and Service Center

884 Pinecrest Dr.

Pineville, LA. 71360

LDH Contact:

Glennen Jones; Glennen.Jones@la.gov

Shannon Todd Aymond; Shannon.Aymond@la.gov; (318) 641-2288

Specification:

The Contractor shall perform the following:

- Remove existing roof membrane and replace any rotted insulation board.

- Install new ½" Structodek HD cover board or equal.

Specify Cover Board Bidding: _____________________

- New HD board shall taper out to edges with a¼" fall minimum.

- All insulation and cover board shall be adhered with Ultraply bonding adhesive or equal.

Specify Bonding Adhesive Bidding: _____________________

- Install and adhere new thermoplastic polyolefin (TPO) membrane to board with adhesive

- Install new TPO unsupported flashing and secure with aluminum strip.

- Install any related curb flashings

- Install any related wood blocking around perimeter edge.

Terms and Conditions:

- Work area shall be kept clean and orderly throughout the project by the Contractor.

- All debris, fasteners, and scrap material shall be removed from the site upon completion by the Contractor.

  - All materials shall be new, of commercial quality, and installed per manufacturer’s specifications.

  - The Contractor shall be responsible for disposal and storage of all materials being removed or installed throughout the project.