Attachment B - Specifications

RFx No.: 3000026665	Title: Forklifts – LADF

Specifications for Food Distribution Forklifts

Brand: Yale or Approved Equal

Model: ERP030VT or Approved Equal

Model Information:

* Counterbalanced Sit-Down Electric Rider Lift Truck 3,000 lb.

* Nominal Capacity @ 24” Load Center

Construction:

* Key Switch Start

* Cooler / Freezer Package – Operating Temperatures -20 ℉ to +120 ℉

* 36 Volt

Powertrain Options:

* UL Classification Type “E”

Battery Compartment:

* 21.5” Battery Compartment

* Battery Compartment With Auxiliary Counterweight Spacer Platform

* Battery Removal without Rollers – Lift Out

Mast:

* 3 Stage Full Free Lift Mast – Class II – 187” (4750mm) Maximum Lift Height – 82” (2080mm) Overall Lowered Height – 59” (1505mm) Free Lift Height without Load Backrest – 236” (5980mm) Overall Extended Height With Load Backrest

* Mast Tilt – 5° Forward / 5° Back

Carriages & Attachments:

* 38.5” (977mm) Wide – Class II – Hook Type Integral Sideshift Carriage

* 48” (1220mm) High Load Backrest Extension

Forks:

* 42” Long x 1.6” Thick x 3.9” Wide (1067mm x 40mm x 100mm) – Class II Hook Type – Standard Taper Forks

Hydraulics:

* 3 Function (1 Auxiliary) Hydraulic Control Valve

* Mechanical Lever Hydraulic Controls – Seat Side – With Return To Set Tilt

Hoses:

* 1 Auxiliary Function Hose Group – 2 Hoses Internally Mounted

Page 1 of 3

Wheels & Tires:

* Drive Tires – 18 x 7-8 – Pneumatic Shaped Solid (PSS) – Standard Compound – Lug Tread

* Steer Tires – 15 x 4.5 x 8 – Pneumatic shaped solid (PSS) – Standard Compound

Operator Compartment:

* 81.5” (2070mm) Overhead Guard – Grid Style

* Mirrors – Dual Side View – Mounted On The Left And Right Side Of Overhead Guard

* Display – High Mount – Mounted In Upper Right Corner Of Overhead Guard

* Fire Extinguisher – 2.5 lb. Dry Chemical

* Rear Drive Handle With Horn Button

Directional Control:

* Seat Side Directional Control

* On Demand Hydrostatic Power Assist Steering

* Steering wheel With Spinner Knob

* Steer Column With Infinitely Adjustable Tilt

Seat:

* Non-Suspension Vinyl Seat

* Seat Belt – Black – No-Cinch With ELR (Emergency Locking Retractor)

Telemetry:

* Yale Vision Wireless Monitoring with Cellular connectivity – GSM or Approved Equal

* 7 Year, Level 1 Telematic Communication and Portal Fees – Monitoring

Lights & Alarms:

* Work Lights – LED Plastic Lens – 2 Front and 1 Rear – Rear Combination

Lights – LED – 2 Brake / Tail / Back-Up

* Audible Alarm – Reverse Direction Activated – Self Adjusting 82 – 102 dB(A)

* Amber Strobe Light – Ignition Activated

* DC to DC Converter – Primary – For Factory Installed Options – 25 Amps / 300 Watts @ 12 Volts

Warranty:

* 12 Months / 2,000 Hours Manufacturer’s Warranty, 36 Months / 6,000 Hours  Manufacturer’s Powertrain Warranty

Literature & Nameplate:

* English Literature Pack and Labels

Battery & Charger Accessories:

* Battery Charging System – Conventional – Includes Single Battery Connector Under The Hood

* Lead Acid Battery Power – Truck Configured To Operate On Lead Acid Battery Power. Not   To Be Used With Lithium-Ion

* Battery Connector – Grey SB® 350

Page 2 of 3

Additional Items or Accessories:

* EZF Watering System

* Direct Fill Link

* Battery Smart Water Monitor

* Battery Charger: 36-volt, 3-Phase 700AH Black Diamond Battery Charger, compatible with 208/240/480 VAC input voltage and capable of charging batteries to a maximum capacity of 700 amp-hours (AH). (Or Approved Equal)

* Battery: 680AH/23.6232KWH Concentric Lead Acid Battery – 5 Rated Service Life – Installed and comes with a 5 - year warranty (Or Approved Equal)

To Be Delivered To:

Louisiana Department of Agriculture & Forestry

5825 Florida Blvd.

Baton Rouge, LA 70806

Page 3 of 3