**If bidding other than specified, please provide manufacturer’s specs for “or equal” items. **

Scope of Work:

The Contractor shall provide all labor, equipment, and materials needed to remove and replace approximately 6,000 sq. ft. of roofing for the Louisiana Department of Health (LDH).

Site Visit Contact:

Shannon Aymond, (318) 641-2286

Location:

Pinecrest Supports and Service Center

884 Pinecrest Dr.

Pineville, la. 71360

# Specifications:

The Contractor shall provide the following:

- Remove the existing roof membrane down to gypsum decking.

- Install a new Permaply 28 or Equal base sheet over gypsum decking and anchor with Ultralok or Equal fasteners.

Specify Base Sheet Bidding: _____________

Specify Fasteners Bidding: ______________

- Install new polyiso tapered system sloped to a ¼" fall to perimeter edges of roof. The taper shall have a 1-1/2" minimum start.

- Adhere tapered system with UIA adhesive or equal.

Specify Adhesive Bidding: _____________

- Install new ½" Dexcell FA or Equal cover board over the tapered insulation and adhere with UIA adhesive or equivalent.

Specify Roofing Cover Board Bidding: _____________

Specify Adhesive Bidding: _____________

- Install new 60 mil TPO membrane over Dexcell FA or Equal cover board and adhere with adhesive thoroughly.

Specify Roofing Cover Board Bidding: _____________

- Install any related curb flashings

- Install any related wood blocking around perimeter edge.

General Terms and Conditions:

- The Contractor shall provide a dumpster and ensure all debris is cleaned from the ground around site daily.

- The job must include two years of workmanship warranty and a 20-year Manufacturers NDL Warranty