**If bidding other than specified, please provide manufacturer’s specs for “or equal” items. **

Scope of Work:

The Contractor shall supply all labor, equipment, supplies, and materials to replace 10 detention doors for the Office of Juvenile Justice (OJJ).

Location:

Jetson Center for Youth – Sunshine Dormitory

15200 Scenic Hwy

Baker, LA 70714

Site Visit Contact:

Allen B. Myrick, allen.myrick2@la.gov, (225) 954-2895

Michael Albarez, Michael.albarez@la.gov, (225) 436-1095

Specifications:

The Contractor shall be responsible for the following:

- Inspect the existing door frame.

- Inspect door alignment, operation, and structural condition.

- Inspect and test detention locking mechanisms and related security hardware.

- Adjust doors to ensure proper alignment, operation, and secure closure.

- Inspect and service electronic or mechanical locking components, where applicable.

- Verify proper operation of control mechanisms, interlocks, and associated security components, where applicable.

- Replace damaged or deteriorated hardware with components suitable for detention-facility applications.

- Remove and properly dispose of any replaced materials and debris generated by the work.

- Provide and install 12GA galv carbon wall wraps at the 10 existing cut openings.

- Weld in detention frames to new framed openings, security caulk and fill with mortar Install half lite door, food pass and detention hardware to include per opening

- One Viking V80 or Equal - l Lock Manual Detention Deadbolt

Specify Lock Bidding: _________________

- One Manual snap lock for food passing doors

- One Raised pull

- One Recessed pull

- One LCN 2214 concealed or Equal closer

Specify Closer Bidding: _________________

- Four 4-1/2" SS detention hinges

- One piece of ½" Lexan

General Terms:

- Because the work involves detention-area security equipment, the Contractor shall coordinate all work with designated facility personnel.

- The Contractor personnel shall comply with all facility security procedures and access requirements.

- No security device, locking mechanism, or detention door shall be left in an unsecured or non-operational condition at the conclusion of each workday unless specifically authorized by facility management.

- Any condition that could compromise the safety or security of the facility shall be reported immediately to designated facility personnel.

- The Contractor personnel shall not make unauthorized modifications to the facility's security systems.

- All replacement components shall be compatible with the existing detention door and security systems.

- All materials and replacement components shall be new, commercial-grade, and appropriate for use in a detention environment unless otherwise approved by the facility.

- The Contractor shall verify compatibility of replacement components with the existing door assemblies prior to installation.

- Upon completion of the work, the contractor shall test each of the 10 detention doors to verify:

  - Proper opening and closing.

  - Proper alignment and operation.

  - Proper engagement of locking mechanisms. Proper operation of associated hardware.

  - Proper operation of electronic/mechanical security components, where applicable. Secure closure and locking.

  - Overall safe and reliable operation.

  - All deficiencies identified during final inspection shall be corrected by the Contractor prior to final acceptance.

- The Contractor shall coordinate the work schedule with facility management to minimize disruption to facility operations. All work shall be completed in a professional and workmanlike manner, with the facility remaining secure and operational throughout the project.