Scope of Work:

The Contractor shall furnish all labor, materials, equipment, tools, supervision, and services necessary to install transmission lines for the LA Educational TV Authority (LETA).

Location:

WLPB Channel 25

6677 River Road

Baton Rouge, LA 70820

LETA Contact:

Steve Ringo, (225) 252-7886, sringo@lpb.org

Specification:

The Contractor shall perform the following:

- Remove the existing 4-1/16 inch line hangers

- Transport 600 feet of removed line and hangers to the KLPB site

(3413 Phillips Road Rayne, LA)

- Install 7-1/16 inch transmission supplied by LETA.

  - The current installed Antenna is Electronics Research INC (ERI)

  - Model: CTW19H4-ETSx-25H.

- Adjust fine matcher during the line sweep.

- Provide a rigging plan for the project.

- Confirm the transmission line is airtight using air/nitrogen provided by LETA.

Terms and Conditions:

- The Contractor shall be responsible for field verifying all dimensions and existing site conditions of work to be performed. The Contractor shall inspect and verify information provided herein on existing materials and equipment, etc. is correct and inform Project Manager (PM) of any errors or questions prior to commencement of work.

- The Contractor shall take all necessary precautions to prevent damages during repairs, to include movement and parking of equipment on grass, other improved, or unimproved areas surrounding site.

- The Contractor shall contact, prior to mobilization and demobilization, facility point of contact which will be provided herein.